APPLIED ENGINEERING AND TECHNOLOGY DIRECTORATE

Triana-Scripts NISTAR Instrument 
Levels 1 and 2 Science Data Products
Data Format Control Book (DFCB)

October 10, 2000

Triana-Scripts

NISTAR Instrument

Levels 1 and 2 Science Data Products

Data Format Control Book (DFCB)

October 10, 2000

Prepared under contract GS-35F-4381G

task order number S-39742-G

by Computer Sciences Corporation

Prepared by:

Michael R. Reid	Date
Triana Ground Data System Engineering, CSC

Reviewed by:

Ralph Casasanta	Date
Triana Ground Data System Engineering Lead, CSC

Concurred by:

Dr. Wayne K. Trail	Date
NISTAR Software Lead, Kinema Research & Software

TBD	Date
TBD

George N. Serafino	Date
GES DAAC, GSFC Code 902

Approved by:

Robert J. Schweiss	Date
TSOC Project Manager, GSFC Code 586

Dr. Steve Lorentz	Date
NISTAR Principal Investigator, NIST

Elizabeth Reid	Date
Triana Science Office Representative, UCSD SIO

Steve Geary	Date
Triana Deputy Principal Investigator, UCSD SIO

Robert Smith	Date
Triana I&T Manager, GSFC Code 740

Goddard Space Flight Center
Greenbelt, Maryland

- Preface

This document provides a complete description of the NISTAR instrument  levels 1 and 2 science data products.  These are the final products, which are archived at the Goddard Space Flight Center (GSFC) Earth Sciences (GES) Distributed Active Archive Center (DAAC).  These products are made available to user by the GES DAAC.

This document is under control of the Triana Ground Systems Project Manager. Proposed changes to this document should be submitted to the Triana implementation manager, along with supportive material justifying the proposed change(s). Changes to this document will be made with a document change notice (DCN) or complete revision. Change bars will highlight any revisions from the previous version of this document.

Questions concerning this document and proposed changes should be addressed to the following:

Triana Ground Systems Project Manager
Systems Management Office, Code 581
Goddard Space Flight Center
Greenbelt, Maryland 20771
USA

- Abstract

This document is the Data Format Control Book (DFCB) for the Triana NISTAR instrument  science data distribution products. This document describes the contents and format of Level 1 and 2 science data products generated by the Triana Ground Data System (GDS)from NISTAR instrument  data.  These Level 1 and 2 products are archived in separate files at the GES DAAC.  The Hierarchical Data Format (HDF) version 4.1r3 available from the National Center for Supercomputing Applications (NCSA) is the file format in which these products are stored.  HDF is a self-describing and standard format for writing scientific data.

- Change Information Page

| List of Effective Pages |  |  |  |
| --- | --- | --- | --- |
| Page Number |  | Issue |  |
| All |  | Original |  |
| Document History |  |  |  |
| Document Number | Status/Issue | Publication Date | CCR Number |
| 474-MGMT-0045 | Original | October 2000 | N/A |

- Table of Contents

1	Introduction	1

1.1	Identification	1

1.2	Products Overview	1

1.3	Purpose	2

1.4	Applicable Documents	2

2	Naming Convention for the HDF Product Files	3

3	The Level 1 Products	4

3.1	Product Overview	4

3.2	Data Volumes	4

3.3	Vgroups	4

3.4	The Science Data	5

3.5	The Instrument Engineering Data	13

3.6	Photodiode Irradiances	22

3.6.1	Irradiances	22

3.6.2	Centroid Coordinates	23

3.7	Ground Calibration Data	24

3.7.1	Primary Aperture dimensions	24

3.7.2	Secondary Aperture dimensions	24

3.7.3	Primary and Secondary Aperture Separation	25

3.7.4	Filter Position versus Filter Motor Steps	25

3.7.5	Thermistors resistance versus temperature	26

3.7.6	Positive Temperature Coefficient (PTC) Thermistors	26

3.7.7	Net temperature coefficients for the electronics (Receiver 1)	26

3.7.8	Ambient Temperature Absolute Electronic Scale Corrections	28

3.7.9	Receiver Power Responsivity at 632nm	29

3.7.10	Spectral Irradiance Responsivity	29

3.7.11	Silicon Photodiode BOL Spectral Responsivity (A/W)	30

3.7.12	Silicon Photodiode BOL Dark Current (A)	30

3.7.13	Filter Transmission Curves	30

3.7.14	Shutter Transmission Function	31

3.7.15	Heat Sink to Shutter Coupling Terms	31

3.7.16	Filter to Receiver Coupling Terms	31

3.8	On-Orbit Calibration Data	32

3.8.1	Pointing Corrections With Respect To EPIC	32

3.8.2	Photodiode Dark Current Measurements	33

3.8.3	Shutter Transmission Function	33

3.8.4	Silicon Photodiode Channel Filter Intercomparison	33

3.8.5	Receivers Filter Intercomparison	34

3.8.6	Total Flux Intercomparison Between All Four Channels	34

3.8.7	Cavity Power Lost to Space	35

3.9	The Geolocation Data	36

3.9.1	Triana Ephemeris	36

3.9.2	Spacecraft Attitude Matrices	36

3.9.3	Instrument Orientation Matrices	37

3.9.4	Instrument Attitude Matrices	37

3.9.5	Lunar Ephemeris	37

3.9.6	Subsatellite Location	38

3.9.7	NISTAR View	38

3.9.8	Spacecraft-Sun Angle	39

3.10	Metadata	39

3.10.1	NISTAR Level 1 Product Metadata	39

3.10.2	Metadata Text Format	40

4	The Level 2 Products	41

4.1	Product Overview	41

4.2	Irradiances	41

4.2.1	Earth Irradiances	41

4.2.2	Lunar Irradiances	43

4.3	Metadata	44

4.3.1	NISTAR Level 2 Product Metadata	45

4.3.2	Metadata Text Format	45

- List of Tables

Table 3-1  Level 1 product Vgroup definitions.	5

Table 3-2  Science Data AppId 82, part 1.	6

Table 3-3  Science Data AppId 82, part 2.	9

Table 3-4  Science Data AppId 82, part 3.	11

Table 3-5  Science Data AppId 86, part 1.	14

Table 3-6  Science Data AppId 86, part 2.	16

Table 3-7  Science Data AppId 86, part 3.	19

Table 3-8  Earth and lunar photodiode irradiances.	22

Table 3-9  Dimension scale for Time dimension in irradiance SDSes.	23

Table 3-10  The photodiode irradiance SDS attributes.	23

Table 3-11  Earth and lunar centroid coordinates.	23

Table 3-12  Dimension scale for SampleTime dimension in centroid coordinates SDSes.	24

Table 3-13  Ground calibration: radii of the primary apertures as measured on the ground.	24

Table 3-14  Ground calibration: radii of the secondary apertures as measured on the ground.	24

Table 3-15  Ground calibration: separation of the primary and secondary apertures as measured on the ground.	25

Table 3-16  The permanent filter positions related to motor step.	25

Table 3-17  Ground calibration: filter position versus motor steps	25

Table 3-18  Ground calibration: laboratory-determined resistance versus temperature curves for the thermistors.	26

Table 3-19  Ground calibration: laboratory-determined resistance versus temperature curves.	26

Table 3-20  Ground calibration: receiver 1 temperature gain and offset for printed wiring assemblies.	27

Table 3-21  Ground calibration: receiver 2 temperature gain and offset for printed wiring assemblies.	27

Table 3-22  Ground calibration: receiver 3 temperature gain and offset for printed wiring assemblies.	27

Table 3-23 Ground calibration: Heat sink temperature gain and offset for printed wiring assemblies.	28

Table 3-24  Ground calibration TBD	28

Table 3-25  Ground calibration: the responsivity (power measurent / power in) for the receivers.	29

Table 3-26  Ground calibration:  responsivity of the receivers versus wavelength of the power source.	29

Table 3-27  Ground calibration: the contains the ratio of the output current in Amps to the input power for versus wavelength.	30

Table 3-28  Ground calibration: dark current (Amps) versus temperature.	30

Table 3-29  Ground calibration: transmission versus wavelength for the 6 filters.	31

Table 3-30  Ground calibration: aperture size versus motor step for the shutters.	31

Table 3-31  Ground calibration: shutter motor step versus temperature offset.	31

Table 3-32  Ground calibration: power versus temperature for each filter over each cavity.	32

Table 3-33  On-Orbit calibration: the pointing orientation with respect to EPIC.	32

Table 3-34  On-Orbit calibration: photodiode dark current measured (in Amps) once a week.	33

Table 3-35  On-Orbit calibration:   aperture versus motor step for the shutter.	33

Table 3-36  On-Orbit calibration: transmission values versus time for each filter for the photodiode.	33

Table 3-37  On-Orbit calibration: transmission versus time for each filter as measured by the three receivers.	34

Table 3-38  On-Orbit calibration:  put the open wheel positions in front of the detectors and measure the total flux (Ampsor Watts).	34

Table 3-39  On-orbit calibration: the power lost to space for each receiver covered by each filter versus time.	35

Table 3-40  Spacecraft ephemeris information.	36

Table 3-41  Spacecraft attitude matrix.	36

Table 3-42  Instrument orientation matrix	37

Table 3-43  Instrument attitude matrix.	37

Table 3-44  Lunar ephemeris.	38

Table 3-45  Subsatellite location.	38

Table 3-46  NISTAR view indication.	38

Table 3-47  Spacecraft Sun angle.	39

Table 3-48  Level 1 product metadata.	39

Table 4-1  Earth irradiances at 10 minute resolution.	41

Table 4-2  Earth irradiances at 1 hour resolution.	42

Table 4-3  Earth irradiances long-term summary.	42

Table 4-4  Lunar irradiances at 10 minute resolution.	43

Table 4-5  Lunar irradiances at 1 hour resolution.	44

Table 4-6  Level 2 product metadata.	45

# Introduction

## Identification

- This document is the Data Format Control Book (DFCB) for the Triana NISTAR instrument  science data products.  It describes the two levels of science products and precisely defines their contents.  Both products are written in the standard Hierarchical Data Format (HDF) using HDF-defined data models and are archived at the GES DAAC.  Information about HDF and official documentation may be found at the HDF web site (http://hdf.ncsa.uiuc.edu).

## Products Overview

- The NISTAR instrument collects irradiance data of the Earth from three radiometer sensors and one photodiode sensor and packs them into the “AppId 82” section of the Triana telemetry downlink.  It also records miscellaneous engineering data and packs them into “AppId 86” of the Triana telemetry downlink.  These data are converted into engineering units and stored as the level 1 NISTAR science data products.  The data are further processed into level 2 products.  These products contain scientifically meaningful data on the Earth’s global temperature and albedo.  From the Triana spacecraft’s unique and distant vantage point, the NISTAR instrument can collect data from nearly the entire sunlit surface of the Earth.  Unlike the products from other prior and current Earth science missions, the Triana NISTAR products contain data for a nearly whole disc image of the Earth at a given moment in time.  NISTAR product files contain data for an entire Earth day.  A day is defined as the interval of time from 00:00:00.00h to 23:59:59.99h UTC.  The level 2 products also contain summary data from previous days’ products in the form of weekly, monthly, and yearly tabulations.  The level 1 and level 2 data products are stored in separate HDF files at the GES DAAC.

- From the Triana spacecraft’s unique vantage point near the Lagrange 1  (L1) point between the Earth and the Sun, the NISTAR instrument can collect data from nearly the entire sunlit surface of the Earth.  Unlike the products from other prior and current Earth science missions, the Triana NISTAR products contain data from sunrise to sunset of the Earth at a given moment in time.

Figure 1  A simulated image of the Earth-Moon system as it might appear from Triana's vantage point.

## Purpose

- This DFCB provides the user with a detailed description of the format and contents of the three Triana NISTAR instrument science data products.  It contains descriptions of the image, location, and other ancillary data and their organization into HDF data objects.  This document is the specification to which the developers of the NISTAR instrument science data processing systems will develop their systems and serves as a guide to end users who will use the data.

## Applicable Documents

- The University of Illinois at Urbana-Champaign, National Center for Supercomputing Applications (NCSA), Getting Started with HDF, Version 3.2, May 1993 (Web URL:  http://hdf.ncsa.uiuc.edu).

- —, HDF User’s Guide, Version 4.1r3, May 1999  (Web URL: http://hdf.ncsa.uiuc.edu).

- —, HDF Reference Manual, Version 4.1r3, May 1999  (Web URL: http://hdf.ncsa.uiuc.edu).

# Naming Convention for the HDF Product Files

All of the data collected by the NISTAR instrument for a given day and all associated ancillary data shall be written to a single HDF file.  The following file naming convention is followed when creating NISTAR level 1 or 2 product files. Each filename shall be thirty characters long and of the form “nist_l_yyyymmdd_aapbbbs_vv.hdf”, where

nist indicates the NISTAR instrument,

l indicates the level of processing: 1 or 2

yyyy indicates the year (e.g., 2002 for the year 2002),

mm indicates the number of the month of the year when obtained (e.g., 04 for April) in UTC,

dd indicates the day of the month (e.g., 07 for the 7th day of the month),

aa indicates the noon-time latitude coordinate of the centroid in degrees (e.g., 37) rounded to the nearest integer

p indicates whether the coordinate is north or south latitude (n for north, s for south),

bbb indicates the noon-time longitude of the centroid in degrees (e.g., 072) rounded to the nearest integer.

s indicates the sign of the coordinate, i.e., whether the longitude is east or west

(e for east, w for west),

vv indicates the version number of the product (range 01…99),

hdf indicates that this is an HDF file.

NISTAR products contain data for a full day.  The centroid values in the file name are the longitude/latitude coordinates for the centroid of the Earth disk as seen from the Triana spacecraft at 12:00:00h UTC.

For example: nist_1_20020407_37n072w_01.hdf.  This is a Level 1 processed NISTAR data product, which was contains data for 07 April 2002 UTC.  It contains Earth data with a noon-time centroid at 37N,72W and is version 01 of the product.

# The Level 1 Products

## Product Overview

Each NISTAR instrument science data product consists of one full (24 hour) day’s worth of data from four sensors, three active cavity radiometers radiometers and a [TBD description] photodiode.  One full day is defined as the interval of time from 00:00:00.00h UTC to 23:59:59.99h UTC.  The data are primarily from the nearly full Earth, but can also contain lunar and star field data.  Ancillary data associated with the science data include data collection time, Earth centroid coordinates (for 12:00.00h UTC), and spacecraft attitude and ephemeris.  The products shall be written using HDF version 4.1r4.  The data objects are grouped into six HDF Vgroups: Science_Data, Engineering_Data, Phototdiode_Irradiances, Ground_Calibration, On-orbit_Calibration, and Geolocation.  [Need to elaborate on this section]

Figure 2  The level 1 HDF product file layout.

## Data Volumes

TBD

## Vgroups

The HDF data objects contained in the NISTAR level 1 data products are organized into Vgroups.  Table 3-1 below defines the Vgroups and specifies the data objects, which are written to each group.  The data objects themselves are described in the later sections of this document.

Table 3-1  Level 1 product Vgroup definitions.

| Vgroup Name | Vgroup Class | Data Object Contents |  | Description |
| --- | --- | --- | --- | --- |
|  |  | Object Name | Type |  |
| Science_Data | Mnemonics | ScienceData_1 | Vdata | Contains the data extracted from AppID 82 of the Triana telemetry. |
|  |  | ScienceData_2 | Vdata |  |
|  |  | ScienceData_3 | Vdata |  |
| Engineering_Data | Mnemonics | InstrumentEngineeringData_1 | Vdata | Contains the data extracted from AppID 86 of the Triana telemetry. |
|  |  | InstrumentEngineeringData_2 | Vdata |  |
|  |  | InstrumentEngineeringData_3 | Vdata |  |
| Photodiode_Irradiances | Irradiances | EarthIrradiances | SDS | Contains irradiances measured by the photodiode and the associated target centroids. |
|  |  | LunarIrradiances | SDS |  |
|  |  | EarthCentroidCoord | SDS |  |
|  |  | LunarCentroidCoords | SDS | Contains the objects, which store the instrument calibrations obtained from ground-based measurements. |
| Ground_Calibration | Calibrations | PrimaryAperatureDimensions | Vdata |  |
|  |  | SecondaryAperatureDimensions | Vdata |  |
|  |  | AperatureSeparations | Vdata |  |
|  |  | FilterPositions | Vdata |  |
|  |  | ThermistorResistance-50to60C | Vdata |  |
|  |  | ThermistorResistance-35to50C | Vdata |  |
|  |  | NetTempCoefReceiver1 | Vdata |  |
|  |  | NetTempCoefReceiver2 | Vdata |  |
|  |  | NetTempCoefReceiver3 | Vdata |  |
|  |  | NetTempCoefHeatSink | Vdata |  |
|  |  | VoltageScaleAdjustments | Vdata |  |
|  |  | ReceiverPowerResponsivity | Vdata |  |
|  |  | SpectralIrradianceResponsivity | Vdata |  |
|  |  | SiliconPhotodiodeResponsivity | Vdata |  |
|  |  | SiliconPhotodiodeDarkCurrent | Vdata |  |
|  |  | FilterTransmissionCurves | Vdata |  |
|  |  | ShutterTransmissionFunction | Vdata |  |
|  |  | HeatSinkToShutterCouplings | Vdata |  |
|  |  | FilterToReceiverCouplings | Vdata |  |
| On-orbit_Calibration | Calibration | InstrumentPointingCorrections | Vdata | Contains the objects, which store the instrument calibrations obtained from on-orbit-based measurements. |
|  |  | PhotodiodeDarkCurrent | Vdata |  |
|  |  | ShutterTransmissionFunction | Vdata |  |
|  |  | PhotodiodeFilterIntercomparison | Vdata |  |
|  |  | ReceiversFilterIntercomparison | Vdata |  |
|  |  | TotalFluxIntercomparison | Vdata |  |
|  |  | CavityPowerLoss | Vdata |  |
| Geolocation | Geolocation | SpacecraftEphemeris | Vdata | Contains the data objects, which store the information needed to geolocate the product science data. |
|  |  | SpacecraftAttitude | Vdata |  |
|  |  | InstrumentOrientationMatrix | Vdata |  |
|  |  | InstrumentAttitudeMatrix | Vdata |  |
|  |  | LunarEphemeris | Vdata |  |
|  |  | SubsatelliteLocation | Vdata |  |
|  |  | NISTARView | Vdata |  |
|  |  | SpacecraftSunAngle | Vdata |  |

## The Science Data

The irradiances collected by the radiometers and photodiode sensors are extracted by the data processing system from AppId 82 of the telemetry received from the Triana spacecraft.  Because of the large sizes of the data sets, they are broken into three parts, each written to a separate HDF Vdata object.  Each data element is directly associated with a data item in AppId 82 of the raw telemetry identified by a mnemonic.  The data in the level 1 products have been converted to engineering units, but retain their one to one associations with the items in the raw telemetry from which they were derived.  Each section of data includes at its beginning the data items from mnemonics H052PVNO…H052SUBS as identifying header.

- Description:	The first third of the level 1 science product from data item NIMJRFRMCNT to NIRC3CMDMTRSTPCT.

- Vgroup:	“Science_Data”

- HDF Structure:	Vdata

- Vdata Name:	“ScienceData_1”

Vdata Class:	“ScienceData”

Size:

Interlace Type:	FULL_INTERLACE

~86,400 records per table, one for each second of data collection per day.

Table 3-2  Science Data AppId 82, part 1.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| H052CNT | DFLOAT_UINT16 | 1 | N/A | 0…16383 | Packet Sequence Control Source Sequence Count |
| H052SECH | TBD | 1 | TBD | TBD | System Time MSWord |
| NIMJRFRMCNT | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | Major Frame Count (Time Reference) |
| NICMDREJCNT | DFNT_UINT8 | 1 | N/A | 0…255 | Command Reject Count |
| NICMDAPTCNT | DFNT_UINT8 | 1 | N/A | 0…255 | Command Accept Count |
| NIPDFLTRTYPE | DFNT_UINT8 | 1 | N/A | 0…3 | Si Photo Diode Filter Type |
| NIRC3FLTRTYPE | DFNT_UINT8 | 1 | N/A | 0…3 | Receiver Cavity 3 Filter Type |
| NIRC2FLTRTYPE | DFNT_UINT8 | 1 | N/A | 0…3 | Receiver Cavity 2 Filter Type |
| NIRC1FLTRTYPE | DFNT_UINT8 | 1 | N/A | 0…3 | Receiver Cavity 1 Filter Type |
| NIAUTOSAFE | DFNT_UINT8 | 1 | N/A | 0…1 | Auto Safe Condition |
| NIARMMODE | DFNT_UINT8 | 1 | N/A | 0…1 | Arm Mode |
| NIINSTMODE | DFNT_UINT8 | 1 | N/A | 0…15 | Instrument Mode |
| NIRC1HTRBIT | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 1 Heater Built In Test |
| NIRC2HTRBIT | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 2 Heater Built In Test |
| NIRC3HTRBIT | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 3 Heater Built In Test |
| NIHSHTRBIT | DFNT_UINT8 | 1 | N/A | 0…1 | Heat Sink Heater Built In Test |
| NIPDBIT | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Built In Test |
| NIQHSSHKBIT | DFNT_UINT8 | 1 | N/A | 0…1 | QHSS Housekeeping Built In Test |
| NIQHSSSCIBIT | DFNT_UINT8 | 1 | N/A | 0…1 | QHSS Science Built In Test |
| NIQHSSMTRBIT | DFNT_UINT8 | 1 | N/A | 0…1 | QHSS Motor Built In Test |
| NITSCBIT | DFNT_UINT8 | 1 | N/A | 0…1 | Triana Spacecraft Computer Built In Test |
| NIFWBIT | DFNT_UINT8 | 1 | N/A | 0…1 | Filter Wheel Built In Test |
| NISHTRSPDBIT | DFNT_UINT8 | 1 | N/A | 0…1 | Shutter Speed Built In Test |
| NIRC3SHTRBIT | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 3 Shutter Built In Test |
| NIRC2SHTRBIT | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 2 Shutter Built In Test |
| NIRC1SHTRBIT | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 1 Shutter Built In Test |
| NI1553BIT | DFNT_UINT8 | 1 | N/A | 0…1 | 1553 Built In Test |
| NIOSBIT | DFNT_UINT8 | 1 | N/A | 0…1 | Operating System Built In Test |
| NIRC1CMDFWMTREN | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Motor Control Command Filter Wheel Motor Enable Status |
| NIRC1CMDPDMTREN | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Motor Control Command Si Photodiode Motor Enable Stat |
| NIRC1CMDRC3MTREN | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Motor Control Command RC3 Motor Enable Status |
| NIRC1CMDRC2MTREN | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Motor Control Command RC2 Motor Enable Status |
| NIRC1CMDRC1MTREN | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Motor Control Command RC1 Motor Enable Status |
| NIRC1CMDFWLDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Motor Control Command Filter Wheel Phase Load Status |
| NIRC1CMDPDLDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Motor Control Command Si Photodiode Phase Load Status |
| NIRC1CMDRC3LDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Motor Control Command RC3 Phase Load Status |
| NIRC1CMDRC2LDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Motor Control Command RC2 Phase Load Status |
| NIRC1CMDRC1LDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Motor Control Command RC1 Phase Load Status |
| NIRC1CMDLDSTPCTR | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Motor Control Command Load Step Counter Status |
| NIRC1CMDSTPMTRST | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Motor Control Command Stop Motor Status |
| NIRC1CMDMTRCTRRS | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Motor Control Command Motor Counter Reset Status |
| NIRC1CMDSTPCLKEN | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Motor Control Command Step Clock Enable Status |
| NIRC1CMDMTRHLDOF | DFNT_UINT8 | 1 | N/A | 0…15 | RC1 Motor Control Command Motor Hold Off |
| NIRC1CMDMTRRVFWD | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Motor Control Command Motor Reverse/Forward Status |
| NIRC1CMDMTRPHSB | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Motor Control Command Motor Phase B Status |
| NIRC1CMDMTRPHSA | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Motor Control Command Motor Phase A Status |
| NIRC1CMDMTRSPD | DFNT_UINT8 | 1 | N/A | 0…7 | RC1 Motor Control Command Motor Speed |
| NIRC1CMDMTRSTPCT | DFNT_UINT16 | 1 | N/A | 0…65535 | RC1 Motor Control Command Motor Step Count |
| NIRC2CMDFWMTREN | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Motor Control Command Filter Wheel Motor Enable Status |
| NIRC2CMDPDMTREN | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Motor Control Command Si Photodiode Motor Enable Status |
| NIRC2CMDRC3MTREN | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Motor Control Command RC3 Motor Enable Status |
| NIRC2CMDRC2MTREN | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Motor Control Command RC2 Motor Enable Status |
| NIRC2CMDRC1MTREN | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Motor Control Command RC1 Motor Enable Status |
| NIRC2CMDFWLDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Motor Control Command Filter Wheel Phase Load Status |
| NIRC2CMDPDLDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Motor Control Command Si Photodiode Phase Load Status |
| NIRC2CMDRC3LDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Motor Control Command RC3 Phase Load Status |
| NIRC2CMDRC2LDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Motor Control Command RC2 Phase Load Status |
| NIRC2CMDRC1LDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Motor Control Command RC1 Phase Load Status |
| NIRC2CMDLDSTPCTR | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Motor Control Command Load Step Counter Status |
| NIRC2CMDSTPMTRST | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Motor Control Command Stop Motor Status |
| NIRC2CMDMTRCTRRS | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Motor Control Command Motor Counter Reset Status |
| NIRC2CMDSTPCLKEN | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Motor Control Command Step Clock Enable Status |
| NIRC2CMDMTRHLDOF | DFNT_UINT8 | 1 | N/A | 0…15 | RC2 Motor Control Command Motor Hold Off |
| NIRC2CMDMTRRVFWD | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Motor Control Command Motor Reverse/Forward Status |
| NIRC2CMDMTRPHSB | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Motor Control Command Motor Phase B Status |
| NIRC2CMDMTRPHSA | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Motor Control Command Motor Phase A Status |
| NIRC2CMDMTRSPD | DFNT_UINT8 | 1 | N/A | 0…7 | RC2 Motor Control Command Motor Speed |
| NIRC2CMDMTRSTPCT | DFNT_UINT16 | 1 | N/A | 0…65535 | RC2 Motor Control Command Motor Step Count |
| NIRC3CMDFWMTREN | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Motor Control Command Filter Wheel Motor Enable Status |
| NIRC3CMDPDMTREN | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Motor Control Command Si Photodiode Motor Enable Status |
| NIRC3CMDRC3MTREN | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Motor Control Command RC3 Motor Enable Status |
| NIRC3CMDRC2MTREN | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Motor Control Command RC2 Motor Enable Status |
| NIRC3CMDRC1MTREN | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Motor Control Command RC1 Motor Enable Status |
| NIRC3CMDFWLDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Motor Control Command Filter Wheel Phase Load Status |
| NIRC3CMDPDLDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Motor Control Command Si Photodiode Phase Load Status |
| NIRC3CMDRC3LDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Motor Control Command RC3 Phase Load Status |
| NIRC3CMDRC2LDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Motor Control Command RC2 Phase Load Status |
| NIRC3CMDRC1LDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Motor Control Command RC1 Phase Load Status |
| NIRC3CMDLDSTPCTR | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Motor Control Command Load Step Counter Status |
| NIRC3CMDSTPMTRST | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Motor Control Command Stop Motor Status |
| NIRC3CMDMTRCTRRS | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Motor Control Command Motor Counter Reset Status |
| NIRC3CMDSTPCLKEN | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Motor Control Command Step Clock Enable Status |
| NIRC3CMDMTRHLDOF | DFNT_UINT8 | 1 | N/A | 0…15 | RC3 Motor Control Command Motor Hold Off |
| NIRC3CMDMTRRVFWD | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Motor Control Command Motor Reverse/Forward Status |
| NIRC3CMDMTRPHSB | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Motor Control Command Motor Phase B Status |
| NIRC3CMDMTRPHSA | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Motor Control Command Motor Phase A Status |
| NIRC3CMDMTRSPD | DFNT_UINT8 | 1 | N/A | 0…7 | RC3 Motor Control Command Motor Speed |
| NIRC3CMDMTRSTPCT | DFNT_UINT16 | 1 | N/A | 0…65535 | RC3 Motor Control Command Motor Step Count |

- Description:	The second third of the level 1 science product from data item NIPDCMDFWMTREN to NIRC1POSOPN.

- Vgroup:	“Science_Data”

- HDF Structure:	Vdata

- Vdata Name:	“ScienceData_2”

Vdata Class:	“ScienceData”

Size:

Interlace Type:	FULL_INTERLACE

~86,400 records per table, one for each second of data collection per day.

Table 3-3  Science Data AppId 82, part 2.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| H052CNT | DFLOAT_UINT16 | 1 | N/A | 0…16383 | Packet Sequence Control Source Sequence Count |
| H052TIME | TBD | 1 | TBD | TBD | System Time when packet was formed |
| NIPDCMDFWMTREN | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Motor Control Command FW Motor Enable Status |
| NIPDCMDPDMTREN | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Motor Control Command SiPD Motor Enable Status |
| NIPDCMDRC3MTREN | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Motor Control Command RC3 Motor Enable Status |
| NIPDCMDRC2MTREN | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Motor Control Command RC2 Motor Enable Status |
| NIPDCMDRC1MTREN | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Motor Control Command RC1 Motor Enable Status |
| NIPDCMDFWLDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Motor Control Command FW Phase Load Status |
| NIPDCMDPDLDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Motor Control Command SiPD Phase Load Status |
| NIPDCMDRC3LDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Motor Control Command RC3 Phase Load Status |
| NIPDCMDRC2LDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Motor Control Command RC2 Phase Load Status |
| NIPDCMDRC1LDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Motor Control Command RC1 Phase Load Status |
| NIPDCMDLDSTPCTR | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Motor Control Command Load Step Counter Status |
| NIPDCMDSTPMTRST | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Motor Control Command Stop Motor Status |
| NIPDCMDMTRCNTRRS | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Motor Control Command Motor Counter Reset Stat |
| NIPDCMDSTPCLCKEN | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Motor Control Command Step Clock Enable Status |
| NIPDCMDMTRHLDOFF | DFNT_UINT8 | 1 | N/A | 0…15 | Si Photodiode Motor Control Command Motor Hold Off |
| NIPDCMDMTRREVFWD | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Motor Control Command Motor Reverse/Frwd Stat |
| NIPDCMDMTRPHSB | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Motor Control Command Motor Phase B Status |
| NIPDCMDMTRPHSA | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Motor Control Command Motor Phase A Status |
| NIPDCMDMTRSPD | DFNT_UINT8 | 1 | N/A | 0…7 | Si Photodiode Motor Control Command Motor Speed |
| NIPDCMDMTRSTPCNT | DFNT_UINT16 | 1 | N/A | 0…65535 | Si Photodiode Motor Control Command Motor Step Count |
| NIFWCMDFWMTREN | DFNT_UINT8 | 1 | N/A | 0…1 | Filter Wheel Motor Control Command FW Motor Enable Status |
| NIFWCMDPDMTREN | DFNT_UINT8 | 1 | N/A | 0…1 | Filter Wheel Motor Control Command SiPD Motor Enable Status |
| NIFWCMDRC3MTREN | DFNT_UINT8 | 1 | N/A | 0…1 | Filter Wheel Motor Control Command RC3 Motor Enable Status |
| NIFWCMDRC2MTREN | DFNT_UINT8 | 1 | N/A | 0…1 | Filter Wheel Motor Control Command RC2 Motor Enable Status |
| NIFWCMDRC1MTREN | DFNT_UINT8 | 1 | N/A | 0…1 | Filter Wheel Motor Control Command RC1 Motor Enable Status |
| NIFWCMDFWLDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | Filter Wheel Motor Control Command FW Phase Load Status |
| NIFWCMDPDLDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | Filter Wheel Motor Control Command SiPD Phase Load Status |
| NIFWCMDRC3LDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | Filter Wheel Motor Control Command RC3 Phase Load Status |
| NIFWCMDRC2LDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | Filter Wheel Motor Control Command RC2 Phase Load Status |
| NIFWCMDRC1LDPHS | DFNT_UINT8 | 1 | N/A | 0…1 | Filter Wheel Motor Control Command RC1 Phase Load Status |
| NIFWCMDLDSTPCTR | DFNT_UINT8 | 1 | N/A | 0…1 | Filter Wheel Motor Control Command Load Step Counter Status |
| NIFWCMDSTPMTRST | DFNT_UINT8 | 1 | N/A | 0…1 | Filter Wheel Motor Control Command Stop Motor Status |
| NIFWCMDMTRCNTRRS | DFNT_UINT8 | 1 | N/A | 0…1 | Filter Wheel Motor Control Command Motor Counter Reset Stat |
| NIFWCMDSTPCLCKEN | DFNT_UINT8 | 1 | N/A | 0…1 | Filter Wheel Motor Control Command Step Clock Enable Status |
| NIFWCMDMTRHLDOFF | DFNT_UINT8 | 1 | N/A | 0…15 | Filter Wheel Motor Control Command Motor Hold Off |
| NIFWCMDMTRREVFWD | DFNT_UINT8 | 1 | N/A | 0…1 | Filter Wheel Motor Control Command Motor Reverse/Frwd Stat |
| NIFWCMDMTRPHSB | DFNT_UINT8 | 1 | N/A | 0…1 | Filter Wheel Motor Control Command Motor Phase B Status |
| NIFWCMDMTRPHSA | DFNT_UINT8 | 1 | N/A | 0…1 | Filter Wheel Motor Control Command Motor Phase A Status |
| NIFWCMDMTRSPD | DFNT_UINT8 | 1 | N/A | 0…7 | Filter Wheel Motor Control Command Motor Speed |
| NIFWCMDMTRSTPCNT | DFNT_UINT16 | 1 | N/A | 0…65535 | Filter Wheel Motor Control Command Motor Step Count |
| NIIRQCOMPLTSTP | DFNT_UINT8 | 1 | N/A | 0…1 | Step Complete Interrupt Request |
| NISTPCLCKENCMD | DFNT_UINT8 | 1 | N/A | 0…1 | Step Clock Enable Command |
| NIMTRDIRCMD | DFNT_UINT8 | 1 | N/A | 0…1 | Motor Direction Command |
| NIMTRHLDOFFCMD | DFNT_UINT8 | 1 | N/A | 0…15 | Motor Hold Off Command |
| NIMTRSPDCMD | DFNT_UINT8 | 1 | N/A | 0…7 | Motor Speed Command |
| NILASTFWPHSB | DFNT_UINT8 | 1 | N/A | 0…1 | Last Filter Wheel Phase B |
| NILASTFWPHSA | DFNT_UINT8 | 1 | N/A | 0…1 | Last Filter Wheel Phase A |
| NILASTPDPHSB | DFNT_UINT8 | 1 | N/A | 0…1 | Last Si Photodiode Phase B |
| NILASTPDPHSA | DFNT_UINT8 | 1 | N/A | 0…1 | Last Si Photodiode Phase A |
| NILASTRC3PHSB | DFNT_UINT8 | 1 | N/A | 0…1 | Last Receiver Cavity 3 Phase B |
| NILASTRC3PHSA | DFNT_UINT8 | 1 | N/A | 0…1 | Last Receiver Cavity 3 Phase A |
| NILASTRC2PHSB | DFNT_UINT8 | 1 | N/A | 0…1 | Last Receiver Cavity 2 Phase B |
| NILASTRC2PHSA | DFNT_UINT8 | 1 | N/A | 0…1 | Last Receiver Cavity 2 Phase A |
| NILASTRC1PHSB | DFNT_UINT8 | 1 | N/A | 0…1 | Last Receiver Cavity 1 Phase B |
| NILASTRC1PHSA | DFNT_UINT8 | 1 | N/A | 0…1 | Last Receiver Cavity 1 Phase A |
| NIFWMTRGOCMD | DFNT_UINT8 | 1 | N/A | 0…1 | Filter Wheel Motor Go Command |
| NIPDMTRGOCMD | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Motor Go Command |
| NIRC3MTRGOCMD | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 3 Motor Go Command |
| NIRC2MTRGOCMD | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 2 Motor Go Command |
| NIRC1MTRGOCMD | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 1 Motor Go Command |
| NIFWPOSCCW | DFNT_UINT8 | 1 | N/A | 0…1 | Filter Wheel Position Counter Clockwise |
| NIFWPOSCW | DFNT_UINT8 | 1 | N/A | 0…1 | Filter Wheel Position Clockwise |
| NIPDPOSCLSD | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Position Closed |
| NIPDPOSOPEN | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Position Open |
| NIRC3POSCLSD | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 3 Position Closed |
| NIRC3POSOPN | DFNT_UINT8 | 1 | N/A | 0…1… | Receiver Cavity 3 Position Open |
| NIRC2POSCLSD | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 2 Position Closed |
| NIRC2POSOPN | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 2 Position Open |
| NIRC1POSCLSD | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 1 Position Closed |
| NIRC1POSOPN | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 1 Position Open |

- Description:	The final third of the level 1 science product from data item NIMTRSTPCNTCMD to NIPDDACAVG10HZ10.

- Vgroup:	“Science_Data”

- HDF Structure:	Vdata

- Vdata Name:	“ScienceData_3”

Vdata Class:	“ScienceData”

Size:

Interlace Type:	FULL_INTERLACE

~86,400 records per table, one for each second of data collection per day.

Table 3-4  Science Data AppId 82, part 3.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| H052CNT | DFLOAT_UINT16 | 1 | N/A | 0…16383 | Packet Sequence Control Source Sequence Count |
| H052TIME | TBD | 1 | TBD | TBD | System Time when packet was formed |
| NIMTRSTPCNTCMD | DFNT_INT16 | 1 | N/A | -32768…32767 | Motor Step Count Command |
| NIPREFWPOSNUM | DFNT_INT16 | 1 | N/A | -32768…32767 | Predicted Filter Wheel Position Number |
| NIRC1PRESHPOSNUM | DFNT_INT16 | 1 | N/A | -32768…32767 | Receiver Cavity 1 Predicted Shutter Position Number |
| NIRC2PRESHPOSNUM | DFNT_INT16 | 1 | N/A | -32768…32767 | Receiver Cavity 2 Predicted Shutter Position Number |
| NIRC3PRESHPOSNUM | DFNT_INT16 | 1 | N/A | -32768…32767 | Receiver Cavity 3 Predicted Shutter Position Number |
| NIPDPRESHPOSNUM | DFNT_INT16 | 1 | N/A | -32768…32767 | Si Photodiode Predicted Shutter Position Number |
| NIRC1BNOMMESAVG | DFNT_INT32 | 1 | N/A | -2^31…(2^31-1) | Receiver Cavity 1 Bridge Null Offset Measurement Measured Avg |
| NIRC1HDACCMDAVG | DFNT_FLOAT32 | 1 | Watts | 0…6.16312E-05 | Receiver Cavity 1 Heater DAC Command Average |
| NIRC1PREDITMDAC | DFNT_UINT16 | 1 | N/A | 0…65535 | Receiver Cavity 1 Pre Dither Multiplying DAC Command |
| *NIRC1HADCMFLAVG | DFNT_FLOAT32 | 1 | Watts | 0…6.85821E-05 | Receiver Cavity 1 Heater ADC Measured Filter Average |
| *NIRC1PTCRERRAVG | DFNT_FLOAT32 | 1 | Ohms | 0…20000 | Receiver Cavity 1 PTC Resistance Error Average |
| *NIRC1PTCMRESAVG | DFNT_FLOAT32 | 1 | Ohms | 0…20000 | Receiver Cavity 1 PTC Measured Resistance Average |
| NIRC1CURRCALTIC | DFNT_UINT8 | 1 | N/A | 0…15 | Receiver Cavity 1 Current Calibration Tick |
| NIRC1DIFCTRLMOD | DFNT_UINT8 | 1 | N/A | 0…7 | Receiver Cavity 1 Differential Control Mode |
| NIRC1ADCCALST | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 1 ADC Calibration Status |
| NIRC1HTRCALST | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 1 Heater Calibration Status |
| NIRC1PTCCALST | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 1 Positive Temp Coeff Calibration Status |
| NIRC1PRECHRGMOD | DFNT_UINT8 | 1 | N/A | 0…3 | Receiver Cavity 1 Precharge Mode |
| NIRC1PTCBRGNLST | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 1 PTC Bridge Nulled Status |
| NIRC1PTCINSATST | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 1 PTC In Saturated Status |
| NIRC1CLSLPCTRL | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 1 Close Loop Control Status |
| NIRC1FZMDACCMD | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 1 Freeze MDAC Command |
| NIRC2BNOMMESAVG | DFNT_INT32 | 1 | N/A | -2^31…(2^31-1) | Receiver Cavity 2 BNOM Measured Avg |
| NIRC2HDACCMDAVG | DFNT_FLOAT32 | 1 | N/A | 0…0.0000590569 | Receiver Cavity 2 Heater DAC Command Average |
| NIRC2PREDITMDAC | DFNT_UINT16 | 1 | N/A | 0…65535 | Receiver Cavity 2 Pre Dither Multiplying DAC Command |
| *NIRC2HADCMFLAVG | DFNT_FLOAT32 | 1 | N/A | 0…7.19234E-05 | Receiver Cavity 2 Heater ADC Measured Filter Average |
| *NIRC2PTCRERRAVG | DFNT_FLOAT32 | 1 | N/A | 0…20000 | Receiver Cavity 2 PTC Resistance Error Average |
| *NIRC2PTCMRESAVG | DFNT_FLOAT32 | 1 | N/A | 0…20000 | Receiver Cavity 2 PTC Measured Resistance Average |
| NIRC2CURRCALTIC | DFNT_UINT8 | 1 | N/A | 0…15 | Receiver Cavity 2 Current Cal Tick |
| NIRC2DIFCTRLMOD | DFNT_UINT8 | 1 | N/A | 0…7 | Receiver Cavity 2 Differential Control Mode |
| NIRC2ADCCALST | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 2 ADC Calibration Status |
| NIRC2HTRCALST | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 2 Heater Calibration Status |
| NIRC2PTCCALST | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 2 Positive Temp Coeff Calibration Status |
| NIRC2PRECHRGMOD | DFNT_UINT8 | 1 | N/A | 0…3 | Receiver Cavity 2 Precharge Mode |
| NIRC2PTCBRGNLST | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 2 PTC Bridge Nulled Status |
| NIRC2PTCINSATST | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 2 PTC In Saturated Status |
| NIRC2CLSLPCTRL | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 2 Close Loop Control Status |
| NIRC2FZMDACCMD | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 2 Freeze MDAC Command |
| NIRC3BNOMMESAVG | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Receiver Cavity 3 BNOM Measured Avg |
| NIRC3HDACCMDAVG | DFNT_FLOAT32 | 1 | N/A | 0…0.0000624692 | Receiver Cavity 3 Heater DAC Command Average |
| NIRC3PREDITMDAC | DFNT_UINT16 | 1 | N/A | 0…65535 | Receiver Cavity 3 Pre Dither Multiplying DAC Command |
| *NIRC3HADCMFLAVG | DFNT_INT32 | 1 | N/A | 0…6.76622E-05 | Receiver Cavity 3 Heater ADC Measured Filter Average |
| *NIRC3PTCRERRAVG | DFNT_FLOAT32 | 1 | N/A | 0…20000 | Receiver Cavity 3 PTC Resistance Error Average |
| *NIRC3PTCMRESAVG | DFNT_FLOAT32 | 1 |  | 0…20000 | Receiver Cavity 3 PTC Measured Resistance Average |
| NIRC3CURRCALTIC | DFNT_UINT8 | 1 | N/A | 0…15 | Receiver Cavity 3 Current Cal Tick |
| NIRC3DIFCTRLMOD | DFNT_UINT8 | 1 | N/A | 0…7 | Receiver Cavity 3 Differential Control Mode |
| NIRC3ADCCALST | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 3 Analog/Digital Converter Calibration Stat |
| NIRC3HTRCALST | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 3 Heater Calibration Status |
| NIRC3PTCCALST | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 3 Positive Temp Coeff Calibration Status |
| NIRC3PRECHRGMOD | DFNT_UINT8 | 1 | N/A | 0…3 | Receiver Cavity 3 Precharge Mode |
| NIRC3PTCBRGNLST | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 3 PTC Bridge Nulled Status |
| NIRC3PTCINSATST | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 3 PTC In Saturated Status |
| NIRC3CLSLPCTRL | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 3 Close Loop Control Status |
| NIRC3FZMDACCMD | DFNT_UINT8 | 1 | N/A | 0…1 | Receiver Cavity 3 Freeze MDAC Command |
| NIHSBNOMMESAVG | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Heat Sink Bridge Null Offset Measurement Measured Average |
| NIHSHDACCMDAVG | DFNT_FLOAT32 | 1 | Watts | 0…3.096072 | Heat Sink Heater Digital/Analog Converter Command Average |
| NIHSPREDITMDAC | DFNT_UINT16 | 1 | N/A | 0…65535 | Heat Sink Pre Dither Multiplying DA Converter Command |
| *NIHSPTCRERRAVG | DFNT_FLOAT32 | 1 | Ohms | 0…20000 | Heat Sink PTC Resistance Error Average |
| *NIHSPTCMRESAVG | DFNT_FLOAT32 | 1 | Ohms | 0…20000 | Heat Sink PTC Measured Resistance Average |
| NIHSCURRCALTIC | DFNT_INT8 | 1 | N/A | 0…15 | Heat Sink Current Cal Tick |
| NIHSADCCALST | DFNT_INT8 | 1 | N/A | 0…1 | Heat Sink Analog/Digital Converter Calibration Status |
| NIHSHTRCALST | DFNT_INT8 | 1 | N/A | 0…1 | Heat Sink Heater Calibration Status |
| NIHSPTCCALST | DFNT_INT8 | 1 | N/A | 0…1 | Heat Sink Positive Temp Coeff Calibration Status |
| NIHSPTCBRGNLST | DFNT_INT8 | 1 | N/A | 0…1 | Heat Sink PTC Bridge Nulled Status |
| NIHSPTCINSATST | DFNT_INT8 | 1 | N/A | 0…1 | Heat Sink PTC In Saturated Statu |
| NIHSCLSLPCTRL | DFNT_INT8 | 1 | N/A | 0…1 | Heat Sink Close Loop Control Status |
| NIHSFZMDACCMD | DFNT_INT8 | 1 | N/A | 0…1 | Heat Sink Freeze MDAC Command |
| NIPDADCAVG10HZ1 | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Si Photodiode ADC Average 10Hz Sample 1 |
| NIPDADCAVG10HZ2 | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Si Photodiode ADC Average 10Hz Sample 2 |
| NIPDADCAVG10HZ3 | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Si Photodiode ADC Average 10Hz Sample 3 |
| NIPDADCAVG10HZ4 | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Si Photodiode ADC Average 10Hz Sample 4 |
| NIPDADCAVG10HZ5 | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Si Photodiode ADC Average 10Hz Sample 5 |
| NIPDADCAVG10HZ6 | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Si Photodiode ADC Average 10Hz Sample 6 |
| NIPDADCAVG10HZ7 | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Si Photodiode ADC Average 10Hz Sample 7 |
| NIPDADCAVG10HZ8 | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Si Photodiode ADC Average 10Hz Sample 8 |
| NIPDADCAVG10HZ9 | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Si Photodiode ADC Average 10Hz Sample 9 |
| NIPDADCAVG10HZ10 | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Si Photodiode ADC Average 10Hz Sample 10 |
| NIPDDACAVG10HZ1 | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Si Photodiode DAC Average 10Hz Sample 1 |
| NIPDDACAVG10HZ2 | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Si Photodiode DAC Average 10Hz Sample 2 |
| NIPDDACAVG10HZ3 | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Si Photodiode DAC Average 10Hz Sample 3 |
| NIPDDACAVG10HZ4 | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Si Photodiode DAC Average 10Hz Sample 4 |
| NIPDDACAVG10HZ5 | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Si Photodiode DAC Average 10Hz Sample 5 |
| NIPDDACAVG10HZ6 | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Si Photodiode DAC Average 10Hz Sample 6 |
| NIPDDACAVG10HZ7 | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Si Photodiode DAC Average 10Hz Sample 7 |
| NIPDDACAVG10HZ8 | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Si Photodiode DAC Average 10Hz Sample 8 |
| NIPDDACAVG10HZ9 | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Si Photodiode DAC Average 10Hz Sample 9 |
| NIPDDACAVG10HZ10 | DFNT_INT32 | 1 | N/A | -2147483648… 2147483647 | Si Photodiode DAC Average 10Hz Sample 10 |

## The Instrument Engineering Data

The engineering data contain status information about the NISTAR instrument .  They are extracted by the data processing system from AppId 86 of the telemetry received from the Triana spacecraft.  Because of the large sizes of the data sets, they are broken into three parts, each written to a separate HDF Vdata object, but stored along with their ancillary information within the same Vgroup.  Each data element is directly associated with a data item in AppId 86 of the raw telemetry identified by a mnemonic.  The data in the level 1 products have been converted to engineering units, but retain their one to one associations with the items in the raw telemetry from which they were derived.  Each section of data includes at its beginning the data items from mnemonics H052PVNO…H052SUBS as identifying header.

- Description:	The first third of the level 1 instrument engineering data set from data item NIRC1PHAMTRI to NISCTSCOR.

- Vgroup:	“Engineering_Data”

- HDF Structure:	Vdata

- Vdata Name:	“InstrumentEngineeringData_1”

Vdata Class:	“EngineeringData”

Size:

Interlace Type:	FULL_INTERLACE

~86,400 Records per table, one for each second of data collection per day.

Table 3-5  Science Data AppId 86, part 1.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| H056CNT | DFNT_UINT16 | 1 | N/A | 0…16383 | Packet Sequence Control Source Sequence Count |
| H056TIME | TBD | 1 | TBD | TBD | System Time when packet was formed |
| NIRC1PHAMTRI | DFNT_FLOAT32 | 1 | milliamps | 0…510.2368 | RC1 Phase A Motor Current |
| NIRC1PHBMTRI | DFNT_FLOAT32 | 1 | milliamps | 0…510.2368 | RC1 Phase B Motor Current |
| NIRC2PHAMTRI | DFNT_FLOAT32 | 1 | milliamps | 0…510.2368 | RC2 Phase A Motor Current |
| NIRC2PHBMTRI | DFNT_FLOAT32 | 1 | milliamps | 0…510.2368 | RC2 Phase B Motor Current |
| NIRC3PHAMTRI | DFNT_FLOAT32 | 1 | milliamps | 0…510.2368 | RC3 Phase A Motor Current |
| NIRC3PHBMTRI | DFNT_FLOAT32 | 1 | milliamps | 0…510.2368 | RC3 Phase B Motor Current |
| NIPDPHAMTRI | DFNT_FLOAT32 | 1 | milliamps | 0…510.2368 | Si Photodiode Phase A Motor Current |
| NIPDPHBMTRI | DFNT_FLOAT32 | 1 | milliamps | 0…510.2368 | Si Photodiode Phase B Motor Current |
| NIFWPHAMTRI | DFNT_FLOAT32 | 1 | milliamps | 0…510.2368 | Filter Wheel Phase A Motor Current |
| NIFWPHBMTRI | DFNT_FLOAT32 | 1 | milliamps | 0…510.2368 | Filter Wheel Phase B Motor Current |
| NIHSTMP | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Heat Sink Temperature |
| NIRC1MTRTMP | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | RC1 Motor Temperature |
| NIRC2MTRTMP | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | RC2 Motor Temperature |
| NIRC3MTRTMP | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | RC3 Motor Temperature |
| NIPDMTRTMP | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Si Photodiode Motor Temperature |
| NIFWMTRTMP | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Filter Wheel Motor Temperature |
| NIPWA11TMP | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Analog Printed Wiring Assembly 1-1 Temperature |
| NIPWA12TMP | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Analog Printed Wiring Assembly 1-2 Temperature |
| NIPWA13TMP | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Analog Printed Wiring Assembly 1-3 Temperature |
| NIPWA14TMP | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Analog Printed Wiring Assembly 1-4 Temperature |
| NIPWA21TMP | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Analog Printed Wiring Assembly 2-1 Temperature |
| NIPWA22TMP | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Analog Printed Wiring Assembly 2-2 Temperature |
| NIPWA23TMP | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Analog Printed Wiring Assembly 2-3 Temperature |
| NIPWA24TMP | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Analog Printed Wiring Assembly 2-4 Temperature |
| NILVPSTMP | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Low Voltage Power Supply Temperature |
| NITLMPWATMP | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Telemetry Printed Wiring Assembly Temperature |
| NIMTRDRPWATMP | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Motor Driver Printed Wiring Assembly Temperature |
| NIP5VDC | DFNT_FLOAT32 | 1 | Volts | 0…20 | +5 VDC |
| NIP15VDC | DFNT_FLOAT32 | 1 | Volts | 0…40 | +15 VDC |
| NIN15VDC | DFNT_FLOAT32 | 1 | Volts | 0…40 | -15 VDC |
| NIP30VDC | DFNT_FLOAT32 | 1 | Volts | 0…100 | +30 VDC |
| NIRC1PHAMTRIPV | DFNT_FLOAT32 | 1 | milliamps | 0…510.2368 | Peak Value RC1 Phase A Motor Current |
| NIRC1PHBMTRIPV | DFNT_FLOAT32 | 1 | milliamps | 0…510.2368 | Peak Value RC1 Phase B Motor Current |
| NIRC2PHAMTRIPV | DFNT_FLOAT32 | 1 | milliamps | 0…510.2368 | Peak Value RC2 Phase A Motor Current |
| NIRC2PHBMTRIPV | DFNT_FLOAT32 | 1 | milliamps | 0…510.2368 | Peak Value RC2 Phase B Motor Current |
| NIRC3PHAMTRIPV | DFNT_FLOAT32 | 1 | milliamps | 0…510.2368 | Peak Value RC3 Phase A Motor Current |
| NIRC3PHBMTRIPV | DFNT_FLOAT32 | 1 | milliamps | 0…510.2368 | Peak Value RC3 Phase B Motor Current |
| NIPDPHAMTRIPV | DFNT_FLOAT32 | 1 | milliamps | 0…510.2368 | Peak Value Si Photodiode Phase A Motor Current |
| NIPDPHBMTRIPV | DFNT_FLOAT32 | 1 | milliamps | 0…510.2368 | Peak Value Si Photodiode Phase B Motor Current |
| NIFWPHAMTRIPV | DFNT_FLOAT32 | 1 | milliamps | 0…510.2368 | Peak Value Filter Wheel Phase A Motor Current |
| NIFWPHBMTRIPV | DFNT_FLOAT32 | 1 | milliamps | 0…510.2368 | Peak Value Filter Wheel Phase B Motor Current |
| NIHSTMPPV | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Peak Value Heat Sink Temperature |
| NIRC1MTRTMPPV | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Peak Value RC1 Motor Temperature |
| NIRC2MTRTMPPV | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Peak Value RC2 Motor Temperature |
| NIRC3MTRTMPPV | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Peak Value RC3 Motor Temperature |
| NIPDMTRTMPPV | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Peak Value Si Photodiode Motor Temperature |
| NIFWMTRTMPPV | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Peak Value Filter Wheel Motor Temperature |
| NIPWA11TMPPV | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Peak Value Analog Printed Wiring Assembly 1-1 Temperature |
| NIPWA12TMPPV | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Peak Value Analog Printed Wiring Assembly 1-2 Temperature |
| NIPWA13TMPPV | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Peak Value Analog Printed Wiring Assembly 1-3 Temperature |
| NIPWA14TMPPV | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Peak Value Analog Printed Wiring Assembly 1-4 Temperature |
| NIPWA21TMPPV | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Peak Value Analog Printed Wiring Assembly 2-1 Temperature |
| NIPWA22TMPPV | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Peak Value Analog Printed Wiring Assembly 2-2 Temperature |
| NIPWA23TMPPV | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Peak Value Analog Printed Wiring Assembly 2-3 Temperature |
| NIPWA24TMPPV | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Peak Value Analog Printed Wiring Assembly 2-4 Temperature |
| NILVPSTMPPV | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Peak Value Low Voltage Power Supply Temperature |
| NITLMPWATMPPV | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Peak Value Telemetry Printed Wiring Assembly Temperature |
| NIMTRDRPWATMPPV | DFNT_FLOAT32 | 1 | Celsius | -273.2… 530.0643 | Peak Value Motor Driver Printed Wiring Assembly Temperature |
| NIP5VDCPV | DFNT_FLOAT32 | 1 | Volts | 0…20 | Peak Value +5 VDC |
| NIP15VDCPV | DFNT_FLOAT32 | 1 | Volts | 0…40 | Peak Value +15 VDC |
| NIN15VDCPV | DFNT_FLOAT32 | 1 | Volts | 0…40 | Peak Value -15 VDC |
| NIP30VDCPV | DFNT_FLOAT32 | 1 | Volts | 0…100 | Peak Value +30 VDC |
| NIHKADCCAL | DFNT_UINT8 | 1 | N/A | 0…1 | ADC Calibration Cmd Status (Housekeeping Analog Board) |
| NIHKLOCRWDT | DFNT_UINT8 | 1 | N/A | 0…1 | Local Redundant Watch Dog Timer (Housekeeping Analog Board) |
| NIHKLOCWDT | DFNT_UINT8 | 1 | N/A | 0…1 | Local Watch Dog Timer (Housekeeping Analog Board) |
| NIHKBSYAD | DFNT_UINT8 | 1 | N/A | 0…1 | Busy Analog to Digital Converter (Housekeeping Analog Board) |
| NIHKCALMD | DFNT_UINT8 | 1 | N/A | 0…1 | Calibrate Mode (Housekeeping Analog Board) |
| NIHKCNVRTMD | DFNT_UINT8 | 1 | N/A | 0…1 | Conversion Mode (Housekeeping Analog Board) |
| NIHKSTATMD | DFNT_UINT8 | 1 | N/A | 0…1 | Status Mode (Housekeeping Analog Board) |
| NITSKORID | DFNT_UINT8 | 1 | N/A | 0…16 | ID Number of Last Task to Have an Overrun |
| NITSKORCNT | DFNT_UINT16 | 1 | N/A | 0…4096 | Last Task Overrun Count |
| NITSKMGROR | DFNT_UINT8 | 1 | N/A | 0…1 | Task Manager Overrun Status |
| NISCPDOR | DFNT_UINT8 | 1 | N/A | 0…1 | Subsystem Control Si Photodiode Overrun Status |
| NIMMMDOR | DFNT_UINT8 | 1 | N/A | 0…1 | Mission Managemnet Mode Overrun Status |
| NISCSHTROR | DFNT_UINT8 | 1 | N/A | 0…1 | Subsystem Control Shutter Overrun Status |
| NISCHTROR | DFNT_UINT8 | 1 | N/A | 0…1 | Subsystem Control Heater Overrun Status |
| NISCFPGAOR | DFNT_UINT8 | 1 | N/A | 0…1 | Subsystem Control Field Programmable Gate Array Overrun Stat |
| NISCFWOR | DFNT_UINT8 | 1 | N/A | 0…1 | Subsystem Control Filter Wheel Overrun Status |
| NISCTSCOR | DFNT_UINT8 | 1 | N/A | 0…1 | Subsystem Control Triana Spacecraft Computer Overrun Status |

Description:	The second third of the level 1 instrument engineering data set from data item NIRC1SHTROSC to NIRC3FCPRECFLTR.

- Vgroup:	“Engineering_Data”

- HDF Structure:	Vdata

- Vdata Name:	“InstrumentEngineeringData_2”

Vdata Class:	“EngineeringData”

Size:

Interlace Type:	FULL_INTERLACE

~86,400 Records per table, one for each second of data collection per day.

Table 3-6  Science Data AppId 86, part 2.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| H056CNT | DFNT_UINT16 | 1 | N/A | 0…16383 | Packet Sequence Control Source Sequence Count |
| H056TIME | TBD | 1 | TBD | TBD | System Time when packet was formed |
| NIRC1SHTROSC | DFNT_UINT16 | 1 | N/A | 0…65535 | RC1 Shutter Open Switch Counter |
| NIRC1SHTRCSC | DFNT_UINT16 | 1 | N/A | 0…65535 | RC1 Shutter Close Switch Counter |
| NIRC2SHTROSC | DFNT_UINT16 | 1 | N/A | 0…65535 | RC2 Shutter Open Switch Counter |
| NIRC2SHTRCSC | DFNT_UINT16 | 1 | N/A | 0…65535 | RC2 Shutter Close Switch Counter |
| NIRC3SHTROSC | DFNT_UINT16 | 1 | N/A | 0…65535 | RC3 Shutter Open Switch Counter |
| NIRC3SHTRCSC | DFNT_UINT16 | 1 | N/A | 0…65535 | RC3 Shutter Close Switch Counter |
| NIPDSHTROSC | DFNT_UINT16 | 1 | N/A | 0…65535 | Si Photodiode Shutter Open Switch Counter |
| NIPDSHTRCSC | DFNT_UINT16 | 1 | N/A | 0…65535 | Si Photodiode Shutter Close Switch Counter |
| NIFWCWSC | DFNT_UINT16 | 1 | N/A | 0…65535 | Filter Wheel Clockwise Switch Counter |
| NIFWCCWSC | DFNT_UINT16 | 1 | N/A | 0…65535 | Filter Wheel Counter Clockwise Switch Counter |
| NIPDPDDACCMD | DFNT_UINT16 | 1 | N/A | 0…65535 | Si Photodiode Pre dither DAC Command |
| NIPDBRDGNULL | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Bridge Nulled Status |
| NIPDINSAT | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode In Saturation Status |
| NIPDFZDACCMD | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Freeze DAC Command Status |
| NIPDPID2P | DFNT_FLOAT32 | 1 | N/A | 0…10.0 | Si Photodiode Proportional Integral Derivative 2 P |
| NIPDPID2I | DFNT_FLOAT32 | 1 | N/A | 0…1.0 | Si Photodiode Proportional Integral Derivative 2 I |
| NIPDPID2D | DFNT_FLOAT32 | 1 | N/A | 0…10.0 | Si Photodiode Proportional Integral Derivative 2 D |
| NIPDPID2KLP | DFNT_FLOAT32 | 1 | N/A | 0…1.0E8 | Si Photodiode Proportional Integral Derivative 2 K Loop |
| NIPDCALST | DFNT_UINT8 | 1 | N/A | 0…16 | Si Photodiode Calibration State |
| NIRC3CALST | DFNT_UINT8 | 1 | N/A | 0…16 | RC3 Calibration State |
| NIRC2CALST | DFNT_UINT8 | 1 | N/A | 0…16 | RC2 Calibration State |
| NIRC1CALST | DFNT_UINT8 | 1 | N/A | 0…16 | RC1 Calibration State |
| NIFWCALST | DFNT_UINT8 | 1 | N/A | 0…16 | Filter Wheel Calibration State |
| NIRC1AREA | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Area |
| NIRC2AREA | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Area |
| NIRC3AREA | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Area |
| NIPDAREA | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Area |
| *NIRC1PTCRSPCMD | DFNT_INT32 | 1 | N/A | 0…20000 | RC1 PTC Resistance Setpoint Command |
| NIRC1PID2P | DFNT_FLOAT32 | 1 | N/A | 0…10.0 | RC1 Proportional Integral Derivative 2 P |
| NIRC1PID2I | DFNT_FLOAT32 | 1 | N/A | 0…1.0 | RC1 Proportional Integral Derivative 2 I |
| NIRC1PID2D | DFNT_FLOAT32 | 1 | N/A | 0…10.0 | RC1 Proportional Integral Derivative 2 D |
| NIRC1PID2KLP | DFNT_FLOAT32 | 1 | N/A | 0…1.0E8 | RC1 Proportional Integral Derivative 2 K Loop |
| NIRC1BNOMSF | DFNT_FLOAT32 | 1 | N/A | 0…2.0 | RC1 Bridge Null Offset Measurement Scale Factor |
| NIRC1MDACSF | DFNT_FLOAT32 | 1 | N/A | 0…2.0 | RC1 MDAC Scale Factor |
| NIRC1HTRSF | DFNT_FLOAT32 | 1 | N/A | 0…2.0 | RC1 Heater Scale Factor |
| NIRC1DIFFMDSF | DFNT_FLOAT32 | 1 | N/A | 0…10.0 | RC1 Differential Mode Scale Factor |
| NIRC1BNOMOFFST | DFNT_FLOAT32 | 1 | N/A | 0…65535 | RC1 Bridge Null Offset Measurement Offset |
| NIRC1HTROFFST | DFNT_FLOAT32 | 1 | N/A | -2^31…2^31-1 | RC1 Heater Offset |
| NIRC1MDACOFFST | DFNT_FLOAT32 | 1 | N/A | -2^31…2^31-1 | RC1 MDAC Offset |
| NIRC1CMDOLPWR | DFNT_FLOAT32 | 1 | Watts | 0… 0.0000616312 | RC1 Commanded Open Loop Power |
| NIRC1SINWVFRQ | DFNT_FLOAT32 | 1 | Hz | 35…155 | RC1 Sine Wave Frequency |
| NIRC1AOPREAFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC1 Auto Open Precharge Type A Filter |
| NIRC1AOPREBFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC1 Auto Open Precharge Type B Filter |
| NIRC1AOPRECFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC1 Auto Open Precharge Type C Filter |
| NIRC1ACPREAFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC1 Auto Close Precharge Type A Filter |
| NIRC1ACPREBFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC1 Auto Close Precharge Type B Filter |
| NIRC1ACPRECFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC1 Auto Close Precharge Type C Filter |
| NIRC1FOPREAFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC1 Fixed Open Precharge Type A Filter |
| NIRC1FOPREBFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC1 Fixed Open Precharge Type B Filter |
| NIRC1FOPRECFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC1 Fixed Open Precharge Type C Filter |
| NIRC1FCPREAFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC1 Fixed Close Precharge Type A Filter |
| NIRC1FCPREBFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC1 Fixed Close Precharge Type B Filter |
| NIRC1FCPRECFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC1 Fixed Close Precharge Type C Filter |
| NIRC2PTCRSPCMD | DFNT_FLOAT32 | 1 | N/A | 0…20000 | RC2 PTC Resistance Setpoint Command |
| NIRC2PID2P | DFNT_FLOAT32 | 1 | N/A | 0…10.0 | RC2 Proportional Integral Derivative 2 P |
| NIRC2PID2I | DFNT_FLOAT32 | 1 | N/A | 0…1.0 | RC2 Proportional Integral Derivative 2 I |
| NIRC2PID2D | DFNT_FLOAT32 | 1 | N/A | 0…10.0 | RC2 Proportional Integral Derivative 2 D |
| NIRC2PID2KLP | DFNT_FLOAT32 | 1 | N/A | 0…1.0E8 | RC2 Proportional Integral Derivative 2 K Loop |
| NIRC2BNOMSF | DFNT_FLOAT32 | 1 | N/A | 0…2.0 | RC2 Bridge Null Offset Measurement Scale Factor |
| NIRC2MDACSF | DFNT_FLOAT32 | 1 | N/A | 0…2.0 | RC2 MDAC Scale Factor |
| NIRC2HTRSF | DFNT_FLOAT32 | 1 | N/A | 0…2.0 | RC2 Heater Scale Factor |
| NIRC2DIFFMDSF | DFNT_FLOAT32 | 1 | N/A | 0…10.0 | RC2 Differential Mode Scale Factor |
| NIRC2BNOMOFFST | DFNT_FLOAT32 | 1 | N/A | 0…65535 | RC2 Bridge Null Offset Measurement Offset |
| NIRC2HTROFFST | DFNT_FLOAT32 | 1 | N/A | 0…65535 | RC2 Heater Offset |
| NIRC2MDACOFFST | DFNT_FLOAT32 | 1 | N/A | 0…65535 | RC2 MDAC Offset |
| NIRC2CMDOLPWR | DFNT_FLOAT32 | 1 | Watts | 0… 0.0000590569 | RC2 Commanded Open Loop Power |
| NIRC2SINWVFRQ | DFNT_FLOAT32 | 1 | Hz | 35…155 | RC2 Sine Wave Frequency |
| NIRC2AOPREAFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC2 Auto Open Precharge Type A Filter |
| NIRC2AOPREBFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC2 Auto Open Precharge Type B Filter |
| NIRC2AOPRECFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC2 Auto Open Precharge Type C Filter |
| NIRC2ACPREAFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC2 Auto Close Precharge Type A Filter |
| NIRC2ACPREBFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC2 Auto Close Precharge Type B Filter |
| NIRC2ACPRECFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC2 Auto Close Precharge Type C Filter |
| NIRC2FOPREAFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC2 Fixed Open Precharge Type A Filter |
| NIRC2FOPREBFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC2 Fixed Open Precharge Type B Filter |
| NIRC2FOPRECFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC2 Fixed Open Precharge Type C Filter |
| NIRC2FCPREAFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC2 Fixed Close Precharge Type A Filter |
| NIRC2FCPREBFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC2 Fixed Close Precharge Type B Filter |
| NIRC2FCPRECFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC2 Fixed Close Precharge Type C Filter |
| NIRC3PTCRSPCMD | DFNT_FLOAT32 | 1 | Ohms | 0…20000.0 | RC3 PTC Resistance Setpoint Command |
| NIRC3PID2P | DFNT_FLOAT32 | 1 | N/A | 0…10.0 | RC3 Proportional Integral Derivative 2 P |
| NIRC3PID2I | DFNT_FLOAT32 | 1 | N/A | 0…1.0 | RC3 Proportional Integral Derivative 2 I |
| NIRC3PID2D | DFNT_FLOAT32 | 1 | N/A | 0…10.0 | RC3 Proportional Integral Derivative 2 D |
| NIRC3PID2KLP | DFNT_FLOAT32 | 1 | N/A | 0…1.0E8 | RC3 Proportional Integral Derivative 2 K Loop |
| NIRC3BNOMSF | DFNT_FLOAT32 | 1 | N/A | 0…2.0 | RC3 Bridge Null Offset Measurement Scale Factor |
| NIRC3MDACSF | DFNT_FLOAT32 | 1 | N/A | 0…2.0 | RC3 MDAC Scale Factor |
| NIRC3HTRSF | DFNT_FLOAT32 | 1 | N/A | 0…2.0 | RC3 Heater Scale Factor |
| NIRC3DIFFMDESF | DFNT_FLOAT32 | 1 | N/A | 0…10.0 | RC3 Differential Mode Scale Factor |
| NIRC3BNOMOFFST | DFNT_FLOAT32 | 1 | N/A | 0…65535 | RC3 Bridge Null Offset Measurement Offset |
| NIRC3HTROFFST | DFNT_FLOAT32 | 1 | N/A | 0…65535 | RC3 Heater Offset |
| NIRC3MDACOFFST | DFNT_FLOAT32 | 1 | N/A | 0…65535 | RC3 MDAC Offset |
| NIRC3CMDOLPWR | DFNT_FLOAT32 | 1 | Watts | 0… 0.00006246916 | RC3 Commanded Open Loop Power |
| NIRC3SINWVFRQ | DFNT_FLOAT32 | 1 | Hz | 35…155 | RC3 Sine Wave Frequency |
| NIRC3AOPREAFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC3 Auto Open Precharge Type A Filter |
| NIRC3AOPREBFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC3 Auto Open Precharge Type B Filter |
| NIRC3AOPRECFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC3 Auto Open Precharge Type C Filter |
| NIRC3ACPREAFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC3 Auto Close Precharge Type A Filter |
| NIRC3ACPREBFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC3 Auto Close Precharge Type B Filter |
| NIRC3ACPRECFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC3 Auto Close Precharge Type C Filter |
| NIRC3FOPREAFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC3 Fixed Open Precharge Type A Filter |
| NIRC3FOPREBFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC3 Fixed Open Precharge Type B Filter |
| NIRC3FOPRECFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC3 Fixed Open Precharge Type C Filter |
| NIRC3FCPREAFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC3 Fixed Close Precharge Type A Filter |
| NIRC3FCPREBFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC3 Fixed Close Precharge Type B Filter |
| NIRC3FCPRECFLTR | DFNT_UINT32 | 1 | N/A | 0…2^32-1 | RC3 Fixed Close Precharge Type C Filter |

- Description:	The final third of the level 1 instrument engineering data set from data item NIHSPTCRSPCMD to NIMDCHKSUM.

- Vgroup:	“Engineering_Data”

- HDF Structure:	Vdata

- Vdata Name:	“InstrumentEngineeringData_3”

Vdata Class:	“EngineeringData”

Size:

Interlace Type:	FULL_INTERLACE

~86,400 Records per table, one for each second of data collection per day.

Table 3-7  Science Data AppId 86, part 3.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| H056CNT | DFNT_UINT16 | 1 | N/A | 0…16383 | Packet Sequence Control Source Sequence Count |
| H056TIME | TBD | 1 | TBD | TBD | System Time when packet was formed |
| *NIHSPTCRSPCMD | DFNT_FLOAT32 | 1 | Ohms | 0…20000 | Heat Sink PTC Resistance Setpoint Command |
| NIHSPID2P | DFNT_FLOAT32 | 1 | N/A | 0…10.0 | Heat Sink Proportional Integral Derivative 2 P |
| NIHSPID2I | DFNT_FLOAT32 | 1 | N/A | 0…1.0 | Heat Sink Proportional Integral Derivative 2 I |
| NIHSPID2D | DFNT_FLOAT32 | 1 | N/A | 0…10.0 | Heat Sink Proportional Integral Derivative 2 D |
| NIHSPID2KLP | DFNT_FLOAT32 | 1 | N/A | 0…1.0E8 | Heat Sink Proportional Integral Derivative 2 K Loop |
| NIHSBNOMSF | DFNT_FLOAT32 | 1 | N/A | 0…2.0 | Heat Sink Bridge Null Offset Measurement Scale Factor |
| NIHSMDACSF | DFNT_FLOAT32 | 1 | N/A | 0…2.0 | Heat Sink MDAC Scale Factor |
| NIHSDIFFMDSF | DFNT_FLOAT32 | 1 | N/A | 0…2.0 | Heat Sink Differential Mode Scale Factor |
| NIHSBNOMOFFST | DFNT_FLOAT32 | 1 | N/A | 0…65535 | Heat Sink Bridge Null Offset Measurement Offset |
| NIHSMDACOFFST | DFNT_UINT16 | 1 | N/A | 0…65535 | Heat Sink MDAC Offset |
| NIHSCMDOLPWR | DFNT_FLOAT32 | 1 | Watts | 0…3.096072 | Heat Sink Commanded Open Loop Power |
| NIHSSINWVFRQ | DFNT_FLOAT32 | 1 | Hz | 35…155 | Heat Sink Sine Wave Frequency |
| NIRC3HTRCALEN | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Heater Calibration Enabled |
| NIHSPTCCALEN | DFNT_UINT8 | 1 | N/A | 0…1 | Heat Sink PTC Calibration Enabled |
| NIRC3PTCCALEN | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Calibration Enabled |
| NIRC2HTRCALEN | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Heater Calibration Enabled |
| NIRC1HTRCALEN | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Heater Calibration Enabled |
| NIRC2PTCCALEN | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Calibration Enabled |
| NIRC1PTCCALEN | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Calibration Enabled |
| NISCCAL | DFNT_UINT8 | 1 | N/A | 0…1 | Calibration of Science Analog Board |
| NIDACWRITEEN | DFNT_UINT8 | 1 | N/A | 0…1 | DAC Write Enable of Science Analog Board |
| NIRC1SINWVVAL | DFNT_FLOAT32 | 1 | Hz | 35…155 | RC1 Sine Wave Value |
| NIRC2SINWVVAL | DFNT_FLOAT32 | 1 | Hz | 35…155 | RC2 Sine Wave Value |
| NIRC1HDACCMD | DFNT_FLOAT32 | 1 | Watts | 0…0.0000616312 | RC1 HDAC Command (Heater) |
| NIRC2HDACCMD | DFNT_FLOAT32 | 1 | Watts | 0…0.0000590569 | RC2 HDAC Command (Heater) |
| NIRC1MDACCMD | DFNT_UINT16 | 1 | N/A | 0…65535 | RC1 MDAC Command (MDAC) |
| NIRC2MDACCMD | DFNT_UINT16 | 1 | N/A | 0…65535 | RC2 MDAC Command (MDAC) |
| NIRC3SINWVVAL | DFNT_FLOAT32 | 1 | Hz | 35…155 | RC3 Sine Wave Value |
| NIHSSINWVVAL | DFNT_FLOAT32 | 1 | Hz | 35…155 | Heat Sink Sine Wave Value |
| NIRC3HDACCMD | DFNT_FLOAT32 | 1 | Watts | 0…0.00006246916 | RC3 HDAC Command (Heater) |
| NIHSHDACCMD | DFNT_FLOAT32 | 1 | Watts | 0…3.096072 | Heat Sink HDAC Command (Heater) |
| NIPDDACCMD | DFNT_UINT16 | 1 | N/A | 0…65535 | Si Photodiode DAC Command |
| NIRC3MDACCMD | DFNT_UINT16 | 1 | N/A | 0…65535 | RC3 MDAC Command (MDAC) |
| NIHSMDACCMD | DFNT_UINT16 | 1 | N/A | 0…65535 | Heat Sink MDAC Command (MDAC) |
| NISCEXTWDT | DFNT_UINT8 | 1 | N/A | 0…1 | External Watch dog timer (Science Analog Board) |
| NISCLOCWDT | DFNT_UINT8 | 1 | N/A | 0…1 | Local Watch dog timer (Science Analog Board) |
| NISCPDBSY | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode Busy (Science Analog Board) |
| NISCRC3HTRBSY | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Heater Busy (Science Analog Board) |
| NISCHSPTCBSY | DFNT_UINT8 | 1 | N/A | 0…1 | Heat Sink PTC Busy (Science Analog Board) |
| NISCRC3PTCBSY | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 PTC Busy (Science Analog Board) |
| NISCRC2HTRBSY | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Heater Busy (Science Analog Board) |
| NISCRC1HTRBSY | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Heater Busy (Science Analog Board) |
| NISCRC2PTCBSY | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 PTC Busy (Science Analog Board) |
| NISCRC1PTCBSY | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 PTC Busy (Science Analog Board) |
| NISCCNVRTMD | DFNT_UINT8 | 1 | N/A | 0…1 | Convert Mode (Science Analog Board) |
| NISCCALMD | DFNT_UINT8 | 1 | N/A | 0…1 | Calibration Mode (Science Analog Board) |
| NISCSTATMD | DFNT_UINT8 | 1 | N/A | 0…1 | Status Mode (Science Analog Board) |
| NIFPGA10KHZFRCNT | DFNT_UINT8 | 1 | N/A | 0…1 | FPGA 10KHz Frame Count |
| NIRC1BNOMRW | DFNT_UINT8 | 1 | N/A | 0…1 | RC1 Bridge Null Offset Measurement Raw (PTC) |
| NIRC2BNOMRW | DFNT_UINT8 | 1 | N/A | 0…1 | RC2 Bridge Null Offset Measurement Raw (PTC) |
| NIRC1HADCRW | DFNT_UINT8 | 1 | Watts | 0…1 | RC1 HADC Raw (Heater) |
| NIRC2HADCRW | DFNT_UINT8 | 1 | Watts | 0…1 | RC2 HADC Raw (Heater) |
| NIRC3BNOMRW | DFNT_UINT8 | 1 | N/A | 0…1 | RC3 Bridge Null Offset Measurement Raw (PTC) |
| NIHSBNOMRW | DFNT_UINT8 | 1 | N/A | 0…1 | Heat Sink Bridge Null Offset Measurement Raw (PTC) |
| NIRC3HADCRW | DFNT_UINT8 | 1 | Watts | 0…1 | RC3 HADC Raw (Heater) |
| NIPDADCRW | DFNT_UINT8 | 1 | N/A | 0…1 | Si Photodiode ADC Raw |
| NIGNDENCWD15 | DFNT_UINT8 | 1 | N/A | 0…1 | Ground Enabled Command Word 15 Status |
| NIGNDENCWD14 | DFNT_UINT8 | 1 | N/A | 0…1 | Ground Enabled Command Word 14 Status |
| NIGNDENCWD13 | DFNT_UINT8 | 1 | N/A | 0…1 | Ground Enabled Command Word 13 Status |
| NIGNDENCWD12 | DFNT_UINT8 | 1 | N/A | 0…1 | Ground Enabled Command Word 12 Status |
| NIGNDENCWD11 | DFNT_UINT8 | 1 | N/A | 0…1 | Ground Enabled Command Word 11 Status |
| NIGNDENCWD10 | DFNT_UINT8 | 1 | N/A0 | 0…1 | Ground Enabled Command Word 10 Status |
| NIGNDENCWD9 | DFNT_UINT8 | 1 | N/A | 0…1 | Ground Enabled Command Word 9 Status |
| NIGNDENCWD8 | DFNT_UINT8 | 1 | N/A | 0…1 | Ground Enabled Command Word 8 Status |
| NIGNDENCWD7 | DFNT_UINT8 | 1 | N/A | 0…1 | Ground Enabled Command Word 7 Status |
| NIGNDENCWD6 | DFNT_UINT8 | 1 | N/A | 0…1 | Ground Enabled Command Word 6 Status |
| NIGNDENCWD5 | DFNT_UINT8 | 1 | N/A | 0…1 | Ground Enabled Command Word 5 Status |
| NIGNDENCWD4 | DFNT_UINT8 | 1 | N/A | 0…1 | Ground Enabled Command Word 4 Status |
| NIGNDENCWD3 | DFNT_UINT8 | 1 | N/A | 0…1 | Ground Enabled Command Word 3 Status |
| NIGNDENCWD2 | DFNT_UINT8 | 1 | N/A | 0…1 | Ground Enabled Command Word 2 Status |
| NIGNDENCWD1 | DFNT_UINT8 | 1 | N/A | 0…1 | Ground Enabled Command Word 2 Status |
| NIMDADD | DFNT_UINT32 | 1 | N/A | 0… 4294967295 | Memory Dump Address |
| NIMDDW1 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 1 |
| NIMDDW2 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 2 |
| NIMDDW3 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 3 |
| NIMDDW4 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 4 |
| NIMDDW5 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 5 |
| NIMDDW6 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 6 |
| NIMDDW7 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 7 |
| NIMDDW8 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 8 |
| NIMDDW9 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 9 |
| NIMDDW10 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 10 |
| NIMDDW11 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 11 |
| NIMDDW12 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 12 |
| NIMDDW13 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 13 |
| NIMDDW14 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 14 |
| NIMDDW15 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 15 |
| NIMDDW16 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 16 |
| NIMDDW17 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 17 |
| NIMDDW18 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 18 |
| NIMDDW19 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 19 |
| NIMDDW20 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 20 |
| NIMDDW21 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 21 |
| NIMDDW22 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 22 |
| NIMDDW23 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 23 |
| NIMDDW24 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 24 |
| NIMDDW25 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 25 |
| NIMDDW26 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 26 |
| NIMDDW27 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 27 |
| NIMDDW28 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 28 |
| NIMDDW29 | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Data Word 29 |
| NIMDCHKSUM | DFNT_UINT16 | 1 | N/A | 0…65535 | Memory Dump Checksum |

## Photodiode Irradiances

These values are the Earth or Moon’s irradiance values as measured by the NISTAR instrument’s photodiode sensor.  The epoch times are expressed in the number of seconds since 01 January 2000, 00:00.00h UTC accurate to 0.01 seconds.  Lunar irradiances and centroid coordinates are included only in products that contain lunar data.  The irradiance and centroid data are scaled to NISTAR EPIC time using HDF dimension scaling.

Dimension Scaling:  The Time dimension of the irradiance SDSes given in Table 3-8 is scaled to 0.1 seconds as described in Table 3-9.  The SampleTime dimension of the Earth and lunar centroid coordinates SDSes given in Table 3-11 is scaled to the same time interval, but by an increment of ten seconds, as described in Table 3-12.  The HDF function Sdsetdimscale or the equivalent Fortran function sfsdscale are used to set the dimension scaling.  See section 3.8.4 of the HDF 4.1r3 User’s Guide for a description of dimension scaling and these functions.

### Irradiances

- Description:	These data sets contain the Earth and lunar irradiances as measured by the photodiode at 0.1 second samplings.

- Vgroup:	“Photodiode_Irradiances”

- HDF Structure:	Scientific Data Set (SDS)

- HDF Tag:		DFTAG_NDG

- Rank:		2

- Dimension 0 Name:	“Time”

- Dimension 0 order:	864,000 (max)

- Dimension Scale:	Scaled to Time in 0.1 second increments

- Size:		6,912,000 bytes (max)

Attributes:	See section 3.6.1.1

Compression:	GZIP deflation (COMP_CODE_DEFLATE), level 6

Table 3-8  Earth and lunar photodiode irradiances.

| SDS Name | HDF Data type | Rank | Dimension Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- | --- |
| EarthIrradiances | DFNT_FLOAT64 | 1 | 864,000 | Wcm2 | 0…1E-5 | Earth irradiance values at 0.1 second sampling intervals in watts per square centimeter as measured by the photodiode. |
| LunarIrradiances | DFNT_FLOAT64 | 1 | 864,000 | Wcm2 | 0…1E-5 | Lunar irradiance values at 0.1 second sampling intervals in watts per square centimeter as measured by the photodiode. |

Table 3-9  Dimension scale for Time dimension in irradiance SDSes.

| Associated SDS Dim. Name | HDF Data type | Number of Values | Units | Range | Increment | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Time | DFNT_FLOAT64 | 864,000 | seconds | 0…2^64-1 | 0.1 seconds | Dimension scale for the Time dimension, 0.1 second increment. |

#### SDS Attributes

Each SDS defined in Table 3-8 has the HDF predefined attributes long_name, units, and valid_range attached to it.  long_name is a label, which in this case is the name of the product, e.g., “Earth Photodiode Irradiances”.  units are the units of the data, in this case., “W/cm^2”.  valid_range is a two element value which contains the upper and lower valid range of the data, in this case it is 0.00001, 0.0.  The first two of these attributes can be easily set in one operation using the Sdsetdatastrs function in the C language version of the HDF API or it equivalent, the sfsdtstr function in the Fortran 77 version of the HDF API.  One should set the valid_range attribute using the Sdsetrange function in the C language version of the HDF API or it equivalent, the sfsrange function in the Fortran 77 version of the HDF API.

Table 3-10  The photodiode irradiance SDS attributes.

| Attribute Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| long_name | DFNT_CHAR8 | 12 | N/A | N/A | The name of the product, i.e., “Earth Photodiode Irradiances” or “Lunar Photodiode Irradiances”.. |
| units | DFNT_CHAR8 | 8 | N/A | N/A | The units of the data, i.e, “W/cm^2”. |
| valid_range | DFNT_UINT16 | 2 | N/A | N/A | The range of the data values in the form max, min, i.e., 0.00001, 0.0. |

### Centroid Coordinates

- Description:	This object contains Earth and lunar centroid coordinates, which map to the photodiode irradiance data defined in Table 3-8.  The terrestrial and lunar geographic coordinates map to their respective irradiance data sets with a cardinality of 1:100.  In other words, one coordinate data point maps to each 100 irradiance data points or every 10 seconds of time.

- Vgroup:	“Photodiode_Irradiances”

- HDF Structure:	Scientific Data Set (SDS)

- HDF Tag:		DFTAG_NDG

- Rank:		2

- Dimension 0 Name:	“SampleTime”

- Dimension 0 order:	8,640 (max)

- Size:		69,120 bytes (max)

Attributes:	None

Compression:	GZIP deflation (COMP_CODE_DEFLATE), level 6

Table 3-11  Earth and lunar centroid coordinates.

| SDS Name | HDF Data type | Rank | Dimension Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- | --- |
| EarthCentroidCoord | DFNT_FLOAT32 | 2 | 8,640×2 | decimal degrees | -180.0… 180.0, -90.0… 90.0, | Earth centroid longitude and latitude coordinates (lon, lat). These values have a 1:100 cardinality with those in the EarthIrradiances SDS. |
| LunarCentroidCoord | DFNT_FLOAT32 | 2 | 8,640×2 | decimal degrees | -180.0… 180.0, -90.0… 90.0, | Lunar centroid longitude and latitude coordinates (lon, lat). These values have a 1:100 cardinality with those in the LunarIrradiances SDS. |

Table 3-12  Dimension scale for SampleTime dimension in centroid coordinates SDSes.

| Associated SDS Dim. Name | HDF Data type | Number of Values | Units | Range | Increment | Description |
| --- | --- | --- | --- | --- | --- | --- |
| SampleTime | DFNT_FLOAT64 | 8,640 | seconds | 0…2^64-1 | 10 seconds | Dimension scale for the SampleTime dimension, 10 second increment. |

## Ground Calibration Data

These data are used to calibrate the level 1 science data.  The data in this section has been determined on the ground and will not change over the course of the mission.  Included in this sections is such information as the sizes of apertures, the transmission properties of the filters, and the temperature sensitivity of various of the optical and electronic components.

### Primary Aperture dimensions

- Description:	This object contains the permanent physical sizes of the primary apertures for the four

- detectors.  This data is determined on the ground and does not change.

- Vgroup:	“Ground_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“PrimaryAperatureDimensions”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 1 records per table.

Table 3-13  Ground calibration: radii of the primary apertures as measured on the ground.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| Receiver1Radius | DFNT_FLOAT32 | 1 | cm | 0…1 | Radius of Receiver 1 Primary Aperture |
| Receiver2Radius | DFNT_FLOAT32 | 1 | cm | 0…1 | Radius of Receiver 2 Primary Aperture |
| Receiver3Radius | DFNT_FLOAT32 | 1 | cm | 0…1 | Radius of Receiver 3 Primary Aperture |
| PhotodiodeRadius | DFNT_FLOAT32 | 1 | cm | 0…1 | Radius of Photodiode Primary Aperture |

### Secondary Aperture dimensions

- Description:	This object contains the permanent physical sizes of the secondary apertures for the four

- detectors.  This data is determined on the ground and does not change.

- Vgroup:	“Ground_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“SecondaryAperatureDimensions”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 1  records per table.

Table 3-14  Ground calibration: radii of the secondary apertures as measured on the ground.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| Receiver1Radius | DFNT_FLOAT32 | 1 | cm | 0…1 | Radius of Receiver 1 Secondary Aperture |
| Receiver2Radius | DFNT_FLOAT32 | 1 | cm | 0…1 | Radius of Receiver 2 Secondary Aperture |
| Receiver3Radius | DFNT_FLOAT32 | 1 | cm | 0…1 | Radius of Receiver 3 Secondary Aperture |
| PhotodiodeRadius | DFNT_FLOAT32 | 1 | cm | 0…1 | Radius of Photodiode Secondary Aperture |

### Primary and Secondary Aperture Separation

- Description:	This object contains the distance between the primary and secondary apertures.  This

- distance is the same for all four detectors, and is determined on the ground.

- Vgroup:	“Ground_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“AperatureSeparations”

Vdata Class:	“Calibration”

Interlace Type:	FULL_INTERLACE

Record Size:

Maximum 1  records per table.

Table 3-15  Ground calibration: separation of the primary and secondary apertures as measured on the ground.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| ApertureSeparation | DFNT_FLOAT32 | 1 | cm | 10…20 | Distance between primary and secondary apertures |

### Filter Position versus Filter Motor Steps

- This object contains the permanent filter position versus motor step.  The stepping motor rotates the filter wheel until a set of filters is in place over the detectors (there are 12 filters on the wheel, but only four detectors).  This data is determined on the ground and does not change.  Table 3-16 indicates which filter is in front of which detector for a given motor step.

Table 3-16  The permanent filter positions related to motor step.

| - Motor Step | - Receiver 1 | - Receiver 2 | - Receiver 3 | - Photodiode |
| --- | --- | --- | --- | --- |
| - 002 | - 1C1 | - 4A1 | - 7B1 | - 10A2 |
| - 102 | - 12A5 | - 3A3 | - 6A6 | - 9A4 |
| - 202 | - 11B2 | - 2B3 | - 5C2 | - 8C3 |
| - 302 | - 10A2 | - 1C1 | - 4A1 | - 7B1 |
| - 402 | - 9A4 | - 12A5 | - 3A3 | - 6A6 |
| - 502 | - 8C3 | - 11B2 | - 2B3 | - 5C2 |
| - 602 | - 7B1 | - 10A2 | - 1C1 | - 4A1 |
| - 702 | - 6A6 | - 9A4 | - 12A5 | - 3A3 |
| - 802 | - 5C2 | - 8C3 | - 11B2 | - 2B3 |
| - 902 | - 4A1 | - 7B1 | - 10A2 | - 1C1 |
| - 1002 | - 3A3 | - 6A6 | - 9A4 | - 12A5 |
| 1102 | 2B3 | 5C2 | 8C3 | 11B2 |

- Description:	This object contains the permanent filter position versus motor step.  In the Vdata

- described below in Table 3-17, we use 1=1C1, 2=2B3, 3=3A3, 4=4A1, 5=5C2,

- 6=6A6, 7=7B1, 8=8C3, 9=9A4, 10=10A2, 11=11B2, 12=12A5.

- Vgroup:	“Ground_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“FilterPositions”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 100 records per table.

Table 3-17  Ground calibration: filter position versus motor steps

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| MotorStep | DFNT_UINT16 | 1 | N/A | 0…65535 | Motor step number |
| Receiver1 | DFNT_UINT8 | 1 | N/A | 1…12 | Filter number versus motor step for receiver 1 |
| Receiver2 | DFNT_UINT8 | 1 | N/A | 1…12 | Filter number versus motor step for receiver 2 |
| Receiver3 | DFNT_UINT8 | 1 | N/A | 1…12 | Filter number versus motor step for receiver 3 |
| Photodiode | DFNT_UINT8 | 1 | N/A | 1…12 | Filter number versus motor step for photodiode |

### Thermistors resistance versus temperature

[NOTE: this number is not in our AppIDs—it is somewhere in the spacecraft housekeeping—should be in VC0]

Description:	This object contains the permanent table of resistance versus temperature for the

thermistors at temperatures from –50C to 60C.  These data are determined on the ground

and do not change.

- Vgroup:	“Ground_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“ThermistorResistance-50to60C”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 300 records per table.

Table 3-18  Ground calibration: laboratory-determined resistance versus temperature curves for the thermistors.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| Temperature | DFNT_FLOAT32 | 1 | Degrees C | -100…100 | Temperature of Thermistor |
| ThermistorResistance | DFNT_FLOAT32 | 1 | Ohms | 0…10^7 | temperature curve for thermistors |

### Positive Temperature Coefficient (PTC) Thermistors

- Description:	This object contains the permanent table of resistance versus temperature for

- temperatures between –35C and 50C.  This data is determined on the ground and does not

- change.

- Vgroup:	“Ground_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“ThermistorResistance-35to50C”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 300 records per table.

Table 3-19  Ground calibration: laboratory-determined resistance versus temperature curves.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| Temperature | DFNT_FLOAT32 | 1 | Degrees C | -100…100 | Temperature of Thermistor |
| Receiver1 | DFNT_FLOAT32 | 1 | Ohms | 0…25000 | Electrical Resistance |
| Receiver2 | DFNT_FLOAT32 | 1 | Ohms | 0…25000 | Electrical Resistance |
| Receiver3 | DFNT_FLOAT32 | 1 | Ohms | 0…25000 | Electrical Resistance |
| HeatSink | DFNT_FLOAT32 | 1 | Ohms | 0…25000 | Electrical Resistance |

### Net temperature coefficients for the electronics (Receiver 1)

The four Vdatas defined below describe the net temperature coefficients (gain and offset corrections versus temperature) for the printed wiring assemblies (PWA) to the power measurement circuits for the three receivers and the heat sink.  These data are determined on the ground and do not change.  The Vdatas are identical in structure except that each references a different temperature grid.

- Description:	This object contains the permanent table of gain and offset corrections versus temperature

- of the printed wiring assemblies (PWA’s) to the power measurement circuits for

- Receiver 1.

- Vgroup:	“Ground_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“NetTempCoefReceiver1”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 300 records per table.

Table 3-20  Ground calibration: receiver 1 temperature gain and offset for printed wiring assemblies.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| Temperature | DFNT_FLOAT32 | 1 | Degrees C | -100…100 | Temperature of PWA |
| Receiver1PWAGain | DFNT_FLOAT32 | 1 | N/A | 0…2 | Gain |
| Receiver1PWAOffset | DFNT_FLOAT32 | 1 | N/A | -32768…32767 | Offset |

- Description:	This object contains the permanent table of gain and offset corrections versus temperature

- of the printed wiring assemblies (PWA’s) to the power measurement circuits for

- Receiver 2.

- Vgroup:	“Ground_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“NetTempCoefReceiver2”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 300 records per table.

Table 3-21  Ground calibration: receiver 2 temperature gain and offset for printed wiring assemblies.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| Temperature | DFNT_FLOAT32 | 1 | Degrees C | -100…100 | Temperature of PWA |
| Receiver2PWAGain | DFNT_FLOAT32 | 1 | N/A | 0…2 | Gain |
| Receiver2PWAOffset | DFNT_FLOAT32 | 1 | N/A | -32768…32767 | Offset |

- Description:	This object contains the permanent table of gain and offset corrections versus temperature

- of the printed wiring assemblies (PWA’s) to the power measurement circuits for

- Receiver 3.

- Vgroup:	“Ground_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“NetTempCoefReceiver3”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 300 records per table.

Table 3-22  Ground calibration: receiver 3 temperature gain and offset for printed wiring assemblies.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| Temperature | DFNT_FLOAT32 | 1 | Degrees C | -100…100 | Temperature of PWA |
| Receiver3PWAGain | DFNT_FLOAT32 | 1 | N/A | 0…2 | Gain |
| Receiver3PWAOffset | DFNT_FLOAT32 | 1 | N/A | -32768…32767 | Offset |

- Description:	This object contains the permanent table of gain and offset corrections versus temperature

- of the printed wiring assemblies (PWA’s) to the power measurement circuits for Heat

- Sink.

- Vgroup:	“Ground_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“NetTempCoefHeatSink”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 200 records per table.

Table 3-23 Ground calibration: Heat sink temperature gain and offset for printed wiring assemblies.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| Temperature | DFNT_FLOAT32 | 1 | Degrees C | -100…100 | Temperature of PWA |
| HeatSinkPWAGain | DFNT_FLOAT32 | 1 | N/A | 0…2 | Gain |
| HeatSink2PWAOffset | DFNT_FLOAT32 | 1 | N/A | -32768…32767 | Offset |

### Ambient Temperature Absolute Electronic Scale Corrections

[TBD]

- Description:	This object contains the permanent voltage scale adjustments (gain and offset) that need

- to be made to the instrument data based on the ambient temperature of the instrument.  These data are determined once on the ground in the laboratory. [TBD:  What is the Range??  SAME AS ABOVE]

- Vgroup:	“Ground_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“VoltageScaleAdjustments”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 300 records per table.

Table 3-24  Ground calibration TBD

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| Temperature | DFNT_FLOAT32 | 1 | Degrees Centigrade | -100… 100 | Temperature of PWA |
| Receiver1ADCVoltageScaleGain | DFNT_FLOAT32 | 1 | N/A | 0…2 | Receiver 1 ambient temperature absolute voltage scale gain adjustment |
| Receiver1ADCVoltageScaleOffset | DFNT_FLOAT32 | 1 | N/A | -32768… 32767 | Receiver 1 ambient temperature absolute voltage scale offset adjustment |
| Receiver2ADCVoltageScaleGain | DFNT_FLOAT32 | 1 | N/A | 0…2 | Receiver 2 ambient temperature absolute voltage scale gain adjustment |
| Receiver2ADCVoltageScaleOffset | DFNT_FLOAT32 | 1 | N/A | -32768… 32767 | Receiver 2 ambient temperature absolute voltage scale offset adjustment |
| Receiver3ADCVoltageScaleGain | DFNT_FLOAT32 | 1 | N/A | 0…2 | Receiver 3 ambient temperature absolute voltage scale gain adjustment |
| Receiver3ADCVoltageScaleOffset | DFNT_FLOAT32 | 1 | N/A | -32768… 32767 | Receiver 3 ambient temperature absolute voltage scale offset adjustment |
| Receiver1DACVoltageScaleGain | DFNT_FLOAT32 | 1 | N/A | 0…2 | Receiver 1 ambient temperature absolute voltage scale gain adjustment |
| Receiver1DACVoltageScaleOffset | DFNT_FLOAT32 | 1 | N/A | -32768… 32767 | Receiver 1 ambient temperature absolute voltage scale offset adjustment |
| Receiver2DACVoltageScaleGain | DFNT_FLOAT32 | 1 | N/A | 0…2 | Receiver 2 ambient temperature absolute voltage scale gain adjustment |
| Receiver2DACVoltageScaleOffset | DFNT_FLOAT32 | 1 | N/A | -32768… 32767 | Receiver 2 ambient temperature absolute voltage scale offset adjustment |
| Receiver3DACVoltageScaleGain | DFNT_FLOAT32 | 1 | N/A | 0…2 | Receiver 3 ambient temperature absolute voltage scale gain adjustment |
| Receiver3DACVoltageScaleOffset | DFNT_FLOAT32 | 1 | N/A | -32768… 32767 | Receiver 3 ambient temperature absolute voltage scale offset adjustment |
| HeatSinkDACVoltageScaleGain | DFNT_FLOAT32 | 1 | N/A | 0…2 | Heat Sink ambient temperature absolute voltage scale gain adjustment |
| HeatSinkDACVoltageScaleOffset | DFNT_FLOAT32 | 1 | N/A | -32768… 32767 | Heat Sink ambient temperature absolute voltage scale offset adjustment |

### Receiver Power Responsivity at 632nm

- Description:	This object contains the permanent table of responsivity values vs. incident power for

- receivers 1 through 3.  This data is determined once on the ground in the laboratory.  The responsivity is the ratio of the measured power to the (carefully calibrated) input power.  [TBD:  Does this one have different scales for the independent variables?]

- Vgroup:	“Ground_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“ReceiverPowerResponsivity”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 300 records per table.

Table 3-25  Ground calibration: the responsivity (power measurent / power in) for the receivers.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| IncidentPower | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Incident power in Watts. |
| Receiver1PowerResponsivity | DFNT_FLOAT32 | 1 | N/A | 0…2 | Radius of Receiver 1 power responsivity |
| Receiver2PowerResponsivity | DFNT_FLOAT32 | 1 | N/A | 0…2 | Radius of Receiver 2 power responsivity |
| Receiver3PowerResponsivity | DFNT_FLOAT32 | 1 | N/A | 0…2 | Radius of Receiver 3 power responsivity |

### Spectral Irradiance Responsivity

- Description:	This object contains the permanent table of spectral irradiance responsivity values vs.

- wavelength for receivers 1 through 3.  The responsivity is the  ratio of the measured power to the input power.  In this Vdata that is done as a function of the wavelength of the input.  These data are determined once on the ground in the laboratory.

- Vgroup:	“Ground_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“SpectralIrradianceResponsivity”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 300 records per table.

Table 3-26  Ground calibration:  responsivity of the receivers versus wavelength of the power source.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| IncidentWavelength | DFNT_FLOAT32 | 1 | meters | 0…1E-4 | Incident wavelength in meters. |
| Receiver1SpectralIrradianceResponsivity | DFNT_FLOAT32 | 1 | N/A | 0…2 | Radius of Receiver 1 spectral irradiance responsivity |
| Receiver2SpectralIrradianceResponsivity | DFNT_FLOAT32 | 1 | N/A | 0…2 | Radius of Receiver 2 spectral irradiance responsivity |
| Receiver3SpectralIrradianceResponsivity | DFNT_FLOAT32 | 1 | N/A | 0…2 | Radius of Receiver 3 spectral irradiance responsivity |

### Silicon Photodiode BOL Spectral Responsivity (A/W)

- Description:	This object contains the permanent table of the responsivity (in Amperes per Watt) of the

- Silicon Photodiode as a function of the wavelength of the light for wavelengths between 200 nm and 1 micron.  These data are determined once on the ground in the laboratory.

- [TBD:  What is “BOL”?]

- Vgroup:	“Ground_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“SiliconPhotodiodeResponsivity”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 300 records per table.

Table 3-27  Ground calibration: the contains the ratio of the output current in Amps to the input power for versus wavelength.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| IncidentWavelength | DFNT_FLOAT32 | 1 | meters | 0…1E-4 | Incident wavelength in meters. |
| SiliconPhotodiodeBOLSpectralResponsivity | DFNT_FLOAT32 | 1 | Amps/Watt | 0…1 | Silicon Photodiode BOL Spectral Responsivity |

### Silicon Photodiode BOL Dark Current (A)

- Description:	This object contains the permanent table of the dark current of the photodiode in

- amperes versus temperature (-30 to +50C).  These data are determined once on the ground in the laboratory.

- Vgroup:	“Ground_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“SiliconPhotodiodeDarkCurrent”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 300 records per table.

Table 3-28  Ground calibration: dark current (Amps) versus temperature.

| Field Name | HDF Data Type | Order | Units |  | Range | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Temperature | DFNT_FLOAT32 | 1 |  | Degrees centigrade | -100…100 | Temperature of PWA |
| SiliconPhotodiodeBOLDarkCurrent | DFNT_FLOAT32 | 1 | Amps |  | 0…2E-6 | Silicon Photodiode BOL dark current |

### Filter Transmission Curves

- Description:	This object contains the permanent table of filter transmission curves covering 250

- nanometers to 20 micrometers for each of the six filter pairs (Note that each “filter,” e.g. 1C1 has 2 filters—on for frequency filtering, and one for thermal filtering).  The names in the table correspond to the code xyz where x=wheel position (1-12), y=filter band (A-C) and z= the number of the filter (there are three B filters and 3 C filters, and 6 A slots with NO filter).  Note that the B’s are 200nm to 4 microns, and the C’s are 720nm to 4 microns.  These data are determined once on the ground in the laboratory.  Also note that each filter is fixed permanently in its wheel position, so wheel position 1 always has filter C1 in it.

- Vgroup:	“Ground_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“FilterTransmissionCurves”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 2000 records per table.

Table 3-29  Ground calibration: transmission versus wavelength for the 6 filters.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| IncidentWavelength | DFNT_FLOAT32 | 1 | meters | 0…1E-4 | Incident wavelength in meters. |
| 1C1 | DFNT_FLOAT32 | 1 | N/A | 0…1 | Transmission ratio |
| 2B3 | DFNT_FLOAT32 | 1 | N/A | 0…1 | Transmission ratio |
| 5C2 | DFNT_FLOAT32 | 1 | N/A | 0…1 | Transmission ratio |
| 7B1 | DFNT_FLOAT32 | 1 | N/A | 0…1 | Transmission ratio |
| 8C3 | DFNT_FLOAT32 | 1 | N/A | 0…1 | Transmission ratio |
| 11B2 | DFNT_FLOAT32 | 1 | N/A | 0…1 | Transmission ratio |

### Shutter Transmission Function

- Description:	This object contains the permanent table of aperture size versus motor step for the

- shutter. This data is determined once on the ground in the laboratory.

- Vgroup:	“Ground_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“ShutterTransmissionFunction”

Vdata Class:	“Calibration”

Record_Size:

Interlace Type:	FULL_INTERLACE

Maximum 220 records per table.

Table 3-30  Ground calibration: aperture size versus motor step for the shutters.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| MotorStep | DFNT_UINT16 | 1 | N/A | 0…300 | Motor step count. |
| ShutterApertureTransmission | DFNT_FLOAT32 | 1 | N/A | 0…1 | What fraction (normalized to 1) of the aperture is open versus motor step |

### Heat Sink to Shutter Coupling Terms

- Description:	This object contains the permanent table of power versus temperature offset on the heat

- sink (magnitude of how to correct for this will depend on the temperature of the heat sink).  These data are determined once on the ground in the laboratory.

- Vgroup:	“Ground_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“HeatSinkToShutterCouplings”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 20 records per table.

Table 3-31  Ground calibration: shutter motor step versus temperature offset.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| Temperature | DFNT_FLOAT32 | 1 | Degrees C | -100…100 | Temperature of PWA |
| Shutter1ToHeatSink | DFNT_FLOAT32 | 1 | Watts | -1…1 | Coupling between Heat Sink and Shutter 1 |
| Shutter2ToHeatSink | DFNT_FLOAT32 | 1 | Watts | -1…1 | Coupling between Heat Sink and Shutter 2 |
| Shutter3ToHeatSink | DFNT_FLOAT32 | 1 | Watts | -1…1 | Coupling between Heat Sink and Shutter 3 |
| Shutter4ToHeatSink | DFNT_FLOAT32 | 1 | Watts | -1…1 | Coupling between Heat Sink and Shutter 4 |

### Filter to Receiver Coupling Terms

- Description:	This object contains the permanent table of power versus temperature for each filter

- position.  These data are determined once on the ground in the laboratory.

- Vgroup:	“Ground_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“FilterToReceiverCouplings”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 300 records per table.

Table 3-32  Ground calibration: power versus temperature for each filter over each cavity.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| Temperature | DFNT_FLOAT32 | 1 | Degrees C | -100…100 | Temperature of PWA |
| Receiver1_1C1 | DFNT_FLOAT32 | 1 | Watts | -6E-5…6E-5 | Receiver to filter coupling |
| Receiver1_11B2 | DFNT_FLOAT32 | 1 | Watts | -6E-5…6E-5 | Receiver to filter coupling |
| Receiver1_8C3 | DFNT_FLOAT32 | 1 | Watts | -6E-5…6E-5 | Receiver to filter coupling |
| Receiver1_7B1 | DFNT_FLOAT32 | 1 | Watts | -6E-5…6E-5 | Receiver to filter coupling |
| Receiver1_5C2 | DFNT_FLOAT32 | 1 | Watts | -6E-5…6E-5 | Receiver to filter coupling |
| Receiver1_2B3 | DFNT_FLOAT32 | 1 | Watts | -6E-5…6E-5 | Receiver to filter coupling |
| Receiver2_1C1 | DFNT_FLOAT32 | 1 | Watts | -6E-5…6E-5 | Receiver to filter coupling |
| Receiver2_11B2 | DFNT_FLOAT32 | 1 | Watts | -6E-5…6E-5 | Receiver to filter coupling |
| Receiver2_8C3 | DFNT_FLOAT32 | 1 | Watts | -6E-5…6E-5 | Receiver to filter coupling |
| Receiver2_7B1 | DFNT_FLOAT32 | 1 | Watts | -6E-5…6E-5 | Receiver to filter coupling |
| Receiver2_5C2 | DFNT_FLOAT32 | 1 | Watts | -6E-5…6E-5 | Receiver to filter coupling |
| Receiver2_2B3 | DFNT_FLOAT32 | 1 | Watts | -6E-5…6E-5 | Receiver to filter coupling |
| Receiver3_1C1 | DFNT_FLOAT32 | 1 | Watts | -6E-5…6E-5 | Receiver to filter coupling |
| Receiver3_11B2 | DFNT_FLOAT32 | 1 | Watts | -6E-5…6E-5 | Receiver to filter coupling |
| Receiver3_8C3 | DFNT_FLOAT32 | 1 | Watts | -6E-5…6E-5 | Receiver to filter coupling |
| Receiver3_7B1 | DFNT_FLOAT32 | 1 | Watts | -6E-5…6E-5 | Receiver to filter coupling |
| Receiver3_5C2 | DFNT_FLOAT32 | 1 | Watts | -6E-5…6E-5 | Receiver to filter coupling |
| Receiver3_2B3 | DFNT_FLOAT32 | 1 | Watts | -6E-5…6E-5 | Receiver to filter coupling |

## On-Orbit Calibration Data

These data are used to calibrate the level 1 science data.  These calibration tables are created and modified based on measurements taken while the spacecraft is in operation (as opposed to the gound-based calibration data described in the previous section).

### Pointing Corrections With Respect To EPIC

Description:	This object contains the pointing corrections between the NISTAR and the EPIC instruments in the spacecraft reference frame.  These values are measured once at the beginning of the mission and again when the spacecraft reaches its final destination orbit (so at least 2 records).

- Vgroup:	“On-orbit_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“InstrumentPointingCorrections”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 100 records per table.

Table 3-33  On-Orbit calibration: the pointing orientation with respect to EPIC.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| NISTAREpoch | DFNT_UINT64 | 1 | N/A | 0…2^64-1 | NISTAR Epoch time |
| q0 | DFNT_FLOAT64 | 1 | N/A | 0…1 | q0 quaternion element |
| q1 | DFNT_FLOAT64 | 1 | N/A | 0…1 | q1 quaternion element |
| q2 | DFNT_FLOAT64 | 1 | N/A | 0…1 | q2 quaternion element |
| q3 | DFNT_FLOAT64 | 1 | N/A | 0…1 | q3 quaternion element |

### Photodiode Dark Current Measurements

- Description:	This object contains photodiode dark current measured versus time.  These data are sampled once a week.  This Vdata contains the measurements from the most recent sampling.

- Vgroup:	“On-orbit_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“PhotodiodeDarkCurrent”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 3600 records per table.

Table 3-34  On-Orbit calibration: photodiode dark current measured (in Amps) once a week.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| NISTAREpoch | DFNT_UINT64 | 1 | N/A | 0…2^64-1 | NISTAR Epoch time |
| PhotodiodeDarkCurrent | DFNT_FLOAT32 | 1 | Amps | 0…6E-5 | Current when photodiode is looking at the back of the shutter. |

### Shutter Transmission Function

- Description:	This object contains a table of aperture versus motor step for the shutter.  These values are measured once at the beginning of the mission and again when the spacecraft reaches its final destination orbit (so at least 2 records).

- Vgroup:	“On-orbit_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“ShutterTransmissionFunction”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 100 records per table.

Table 3-35  On-Orbit calibration:   aperture versus motor step for the shutter.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| MotorStep | DFNT_UINT16 | 1 | N/A | 0…300 | Motor step count. |
| ShutterApertureTransmission | DFNT_FLOAT32 | 1 | N/A | 0…1 | What fraction (normalized to 1) of the aperture is open versus motor step |

### Silicon Photodiode Channel Filter Intercomparison

- Description:	This object contains a table of transmission values versus time for each filter as measured by the silicon photodiode channel.  [Weekly]

- Vgroup:	“On-orbit_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“PhotodiodeFilterIntercomparison”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 3600 records per table.

Table 3-36  On-Orbit calibration: transmission values versus time for each filter for the photodiode.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| NISTAREpoch | DFNT_UINT64 | 1 | N/A | 0…2^64-1 | NISTAR Epoch time |
| Filter_1C1 | DFNT_FLOAT32 | 1 | N/A | 0…1 | Transmission reading through filter by photodiode |
| Filter _2B3 | DFNT_FLOAT32 | 1 | N/A | 0…1 | Transmission reading through filter by photodiode |
| Filter _5C2 | DFNT_FLOAT32 | 1 | N/A | 0…1 | Transmission reading through filter by photodiode |
| Filter _7B1 | DFNT_FLOAT32 | 1 | N/A | 0…1 | Transmission reading through filter by photodiode |
| Filter _8C3 | DFNT_FLOAT32 | 1 | N/A | 0…1 | Transmission reading through filter by photodiode |
| Filter _11B2 | DFNT_FLOAT32 | 1 | N/A | 0…1 | Transmission reading through filter by photodiode |

### Receivers Filter Intercomparison

- Description:	This object contains a table of transmission values versus time for each filter as measured by the three receivers.

- Vgroup:	“On-orbit_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“ReceiversFilterIntercomparison”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 3600 records per table.

Table 3-37  On-Orbit calibration: transmission versus time for each filter as measured by the three receivers.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| NISTAREpoch | DFNT_UINT64 | 1 | N/A | 0…2^64-1 | NISTAR Epoch time |
| Receiver1_1C1 | DFNT_FLOAT32 | 1 | N/A | 0...1 | Transmission through filter by receiver 1 |
| Receiver1_2B3 | DFNT_FLOAT32 | 1 | N/A | 0...1 | Transmission through filter by receiver 1 |
| Receiver1_5C2 | DFNT_FLOAT32 | 1 | N/A | 0...1 | Transmission through filter by receiver 1 |
| Receiver1_7B1 | DFNT_FLOAT32 | 1 | N/A | 0...1 | Transmission through filter by receiver 1 |
| Receiver1_8C3 | DFNT_FLOAT32 | 1 | N/A | 0...1 | Transmission through filter by receiver 1 |
| Receiver1_11B2 | DFNT_FLOAT32 | 1 | N/A | 0...1 | Transmission through filter by receiver 1 |
| Receiver2_1C1 | DFNT_FLOAT32 | 1 | N/A | 0...1 | Transmission through filter by receiver 2 |
| Receiver2_2B3 | DFNT_FLOAT32 | 1 | N/A | 0...1 | Transmission through filter by receiver 2 |
| Receiver2_5C2 | DFNT_FLOAT32 | 1 | N/A | 0...1 | Transmission through filter by receiver 2 |
| Receiver2_7B1 | DFNT_FLOAT32 | 1 | N/A | 0...1 | Transmission through filter by receiver 2 |
| Receiver2_8C3 | DFNT_FLOAT32 | 1 | N/A | 0...1 | Transmission through filter by receiver 2 |
| Receiver2_11B2 | DFNT_FLOAT32 | 1 | N/A | 0...1 | Transmission through filter by receiver 2 |
| Receiver3_1C1 | DFNT_FLOAT32 | 1 | N/A | 0...1 | Transmission through filter by receiver 3 |
| Receiver3_2B3 | DFNT_FLOAT32 | 1 | N/A | 0...1 | Transmission through filter by receiver 3 |
| Receiver3_5C2 | DFNT_FLOAT32 | 1 | N/A | 0...1 | Transmission through filter by receiver 3 |
| Receiver3_7B1 | DFNT_FLOAT32 | 1 | N/A | 0...1 | Transmission through filter by receiver 3 |
| Receiver3_8C3 | DFNT_FLOAT32 | 1 | N/A | 0...1 | Transmission through filter by receiver 3 |
| Receiver3_11B2 | DFNT_FLOAT32 | 1 | N/A | 0...1 | Transmission through filter by receiver 3 |

### Total Flux Intercomparison Between All Four Channels

- Description:	This object contains a table of transmission values versus time for each filter as measured by the photodiode channel.

- Vgroup:	“On-orbit_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“TotalFluxIntercomparison”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 3600 records per table.

Table 3-38  On-Orbit calibration:  put the open wheel positions in front of the detectors and measure the total flux (Ampsor Watts).

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| NISTAREpoch | DFNT_UINT32 | 1 | N/A | 0…2^64-1 | NISTAR Epoch time |
| Photodiode_3A3 | DFNT_FLOAT32 | 1 | Amps | 0…2E-6 | Power reading through filter by photodiode |
| Photodiode _4A1 | DFNT_FLOAT32 | 1 | Amps | 0…2E-6 | Power reading through filter by photodiode |
| Photodiode _6A6 | DFNT_FLOAT32 | 1 | Amps | 0…2E-6 | Power reading through filter by photodiode |
| Photodiode _9A4 | DFNT_FLOAT32 | 1 | Amps | 0…2E-6 | Power reading through filter by photodiode |
| Photodiode _10A2 | DFNT_FLOAT32 | 1 | Amps | 0…2E-6 | Power reading through filter by photodiode |
| Photodiode _12A5 | DFNT_FLOAT32 | 1 | Amps | 0…2E-6 | Power reading through filter by photodiode |
| Receiver1_3A3 | DFNT_FLOAT32 | 1 | Watts | 0…1E-5 | Power reading through filter by receiver 1 |
| Receiver1_4A1 | DFNT_FLOAT32 | 1 | Watts | 0…1E-5 | Power reading through filter by receiver 1 |
| Receiver1_6A6 | DFNT_FLOAT32 | 1 | Watts | 0…1E-5 | Power reading through filter by receiver 1 |
| Receiver1_9A4 | DFNT_FLOAT32 | 1 | Watts | 0…1E-5 | Power reading through filter by receiver 1 |
| Receiver1_10A2 | DFNT_FLOAT32 | 1 | Watts | 0…1E-5 | Power reading through filter by receiver 1 |
| Receiver1_12A5 | DFNT_FLOAT32 | 1 | Watts | 0…1E-5 | Power reading through filter by receiver 1 |
| Receiver2_3A3 | DFNT_FLOAT32 | 1 | Watts | 0…1E-5 | Power reading through filter by receiver 2 |
| Receiver2_4A1 | DFNT_FLOAT32 | 1 | Watts | 0…1E-5 | Power reading through filter by receiver 2 |
| Receiver2_6A6 | DFNT_FLOAT32 | 1 | Watts | 0…1E-5 | Power reading through filter by receiver 2 |
| Receiver2_9A4 | DFNT_FLOAT32 | 1 | Watts | 0…1E-5 | Power reading through filter by receiver 2 |
| Receiver2_10A2 | DFNT_FLOAT32 | 1 | Watts | 0…1E-5 | Power reading through filter by receiver 2 |
| Receiver2_12A5 | DFNT_FLOAT32 | 1 | Watts | 0…1E-5 | Power reading through filter by receiver 2 |
| Receiver3_3A3 | DFNT_FLOAT32 | 1 | Watts | 0…1E-5 | Power reading through filter by receiver 3 |
| Receiver3_4A1 | DFNT_FLOAT32 | 1 | Watts | 0…1E-5 | Power reading through filter by receiver 3 |
| Receiver3_6A6 | DFNT_FLOAT32 | 1 | Watts | 0…1E-5 | Power reading through filter by receiver 3 |
| Receiver3_9A4 | DFNT_FLOAT32 | 1 | Watts | 0…1E-5 | Power reading through filter by receiver 3 |
| Receiver3_10A2 | DFNT_FLOAT32 | 1 | Watts | 0…1E-5 | Power reading through filter by receiver 3 |
| Receiver3_12A5 | DFNT_FLOAT32 | 1 | Watts | 0…1E-5 | Power reading through filter by receiver 3 |

### Cavity Power Lost to Space

[TBD:  This section was called “Space-View Calibration”.]

- Description:	This object contains a table of the amount of power the cavity loses to space.

- Vgroup:	“On-orbit_Calibration”

- HDF Structure:	Vdata

- Vdata Name:	“CavityPowerLoss”

Vdata Class:	“Calibration”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum 500 records per table.

Table 3-39  On-orbit calibration: the power lost to space for each receiver covered by each filter versus time.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| NISTAREpoch | DFNT_UINT64 | 1 | N/A | 0…2^64-1 | NISTAR Epoch time |
| Receiver1_1C1 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver1_2B3 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver1_3A3 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver1_4A1 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver1_5C2 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver1_6A6 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver1_7B1 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver1_8C3 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver1_9A4 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver1_10A2 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver1_11B2 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver1_12A5 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver2_1C1 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver2_2B3 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver2_3A3 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver2_4A1 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver2_5C2 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver2_6A6 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver2_7B1 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver2_8C3 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver2_9A4 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver2_10A2 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver2_11B2 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver2_12A5 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver3_1C1 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver3_2B3 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver3_3A3 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver3_4A1 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver3_5C2 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver3_6A6 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver3_7B1 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver3_8C3 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver3_9A4 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver3_10A2 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver3_11B2 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |
| Receiver3_12A5 | DFNT_FLOAT32 | 1 | Watts | 0…1E-4 | Space View reading. |

## The Geolocation Data

The geolocation data are sets of positional, ephemeris, and attitude information, which are used as input to an algorithm, which will compute longitude and latitude coordinates for any given pixel.  The geolocation data consist of 6 Vdatas as described in the following tables.

### Triana Ephemeris

- Description:	Specifies the Triana spacecraft position and velocity in geocentric rectangular inertial

- J2000 coordinates.

- Vgroup:	“Geolocation”

- HDF Structure:	Vdata

- Vdata Name:	“SpacecraftEphemeris”

Vdata Class:	“Geolocation”

Interlace Type:	FULL_INTERLACE

One record per table

Table 3-40  Spacecraft ephemeris information.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| NISTAREpoch | DFNT_UINT64 | 1 | JulianDay | 2451544… 2465442 | Triana quaternian time (J.D.). |
| TrianaEphemerisPosition | DFNT_FLOAT64 | 3 | km | -2E6…+2E6 | x, y, z components of position 1. |
| TrianaEphemerisVelocity | DFNT_FLOAT64 | 3 | km/s | -11…+11 | x, y, z components of velocity. |

### Spacecraft Attitude Matrices

- Description:	Expresses the attitude matrix of the Triana spacecraft as a quaternian.  The attitude matrix

- describes the angular orientation of the spacecraft in geocentric rectangular intertial

- J2000 coordinates.  These data are used to create the rotational matrices for

- calculating the direction that the NISTAR instrument is pointed.

- Vgroup:	“Geolocation”

- HDF Structure:	Vdata

- Vdata Name:	“SpacecraftAttitude”

Vdata Class:	“Geolocation”

Record Size:

Interlace Type:	FULL_INTERLACE

One record per table.

Table 3-41  Spacecraft attitude matrix.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| NISTAREpoch | DFNT_UINT64 | 1 | JulianDay | 2451544…2465442 | Triana quaternian time (J.D.). |
| AttitudeQuaternian | DFNT_FLOAT32 | 4 | N/A | -1…+1 | q0, q1, q2, q3 quaternion components of the interpolated spacecraft angular orientation component. |

### Instrument Orientation Matrices

- Description:	The rotation matrix that describes the mechanical alignment between the Star

- Tracker and the NISTAR instrument aboard the Triana spacecraft.  The mechanical and

- the optical axes within the instrument are nearly identical.  The matrix

- transforms the Star Tracker’s coordinates to NISTAR coordinates.  These data make up

- a 3×3 matrix where each field contains the three column values for the row and each

- record in the Vdata is a row of the matrix.  According to the Triana AOCS Hardware

- Coordinate System document, NISTAR points along the +X axis of the Observatory

- Axis System (OAS).

- Vgroup:	“Geolocation”

- HDF Structure:	Vdata

- Vdata Name:	“InstrumentOrientationMatrix”

Vdata Class:	“Geolocation”

Record Size:

Interlace Type:	FULL_INTERLACE

Three records per table.  Each record is a row of the 3×3 matrix.

Table 3-42  Instrument orientation matrix

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| Row | DFNT_UINT8 | 1 | N/A | 1…3 | The matrix row number. |
| OrientationMatrix | DFNT_FLOAT64 | 3 | N/A | -1…+1 | EPIC camera orientation to Star Tracker. Each value contains the three column elements, i.e., r1, r2, r3. |

### Instrument Attitude Matrices

- Description:	The attitude matrix, which describes the pointing direction of the EPIC instrument in

- geocentric rectangular inertial J2000 coordinates at the image exposure time.  These

- data form a 3×3 matrix where each record in the Vdata is a row of its respective

- matrix.  Each field contains the three values for the column of the respective matrix.

- These data indicate the direction that the instrument is pointing.

- Vgroup:	“Geolocation”

- HDF Structure:	Vdata

- Vdata Name:	“InstrumentAttitudeMatrix”

Vdata Class:	“Geolocation”

Record Size:

Interlace Type:	FULL_INTERLACE

Three records per table.

Table 3-43  Instrument attitude matrix.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| Row | DFNT_UINT8 | 1 | N/A | 1…3 | The matrix row number. |
| AttitudeMatrix | DFNT_FLOAT64 | 3 | N/A | -1…+1 | x, y, z components of the EPIC camera attitude matrix at image time for the given row and the given wavelength. |

### Lunar Ephemeris

- Description:	Describes the Moon’s position and velocity in geocentric rectangular inertial J2000

- coordinates.  The first and second predicted ephemerides are the closest available

- values that temporally bracket the image exposure time.  The third ephemeris is an

- interpolation to the image exposure time.

- Vgroup:	“Geolocation”

- HDF Structure:	Vdata

- Vdata Name:	“LunarEphemeris”

Vdata Class:	“Geolocation”

Record Size:

Interlace Type:	FULL_INTERLACE

One record per table.

Table 3-44  Lunar ephemeris.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| NISTAREpoch | DFNT_UINT64 | 1 | JulianDay | 2451544… 2465442 | Triana epoch used in interpolation to image time (J.D.). [TBD: what form, epoch??] |
| LunarEphemerisPosition | DFNT_FLOAT64 | 3 | km | -2E6…+2E6 | x, y, z components of position 1. |
| LunarEphemerisVelocity | DFNT_FLOAT64 | 3 | km/s | -11…+11 | x, y, z components of velocity. |

### Subsatellite Location

- Description:	This object contains the latitude and longitude of the spacecraft’s subsatellite point, i.e.,

- the latitude and longitude of the point on the surface of the Earth through which a straight

- line connecting the center of the Earth and the spacecraft passes.

- Vgroup:	“Geolocation”

- HDF Structure:	Vdata

- Vdata Name:	“SubsatelliteLocation”

Vdata Class:	“Geolocation”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum TBD Records per table.

Table 3-45  Subsatellite location.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| NISTAREpoch | DFNT_UINT64 | 1 | Julian Day | 2451544… 2465442 | Time of the Ephemeris data used in the subsatellite calculation (J.D.). |
| GreenwichHourAngle | DFNT_FLOAT32 | 1 | radians | 0…2 | Angle offset computed by geolocation algorithm. |
| SubsatelliteLongitude | DFNT_FLOAT32 | 1 | radians | -… | Longitude of the subsatellite point as calculated from ephemeris data |
| SubsatelliteLatitude | DFNT_FLOAT32 | 1 | radians | -/2…/2 | Latitude of the subsatellite point as calculated from ephemeris data |

### NISTAR View

- Description:	Information in this object will tell what is in the instrument’s field of view.  In order for NISTAR to take data, the object (e.g. Earth) needs to be within a 1 degree angle of the direction in which the instrument is pointing.  Furthermore, there can be no other objects (e.g. the Moon) within 3.5 degrees of the pointing direction.  The NISTARView parameter below has a value of 1 when the Earth is “in position” and no other objects are in the field.  Similarly, the value is 2 when only the moon is in the field.  3 indicates deep space. 4 indicates a problem (e.g. the earth and the moon are both within the 3.5 degree window.  We are allowing additional values in case a decision is made to trap for certain conditions such as the Moon passing in front of the Earth.

- Vgroup:	“Geolocation”

- HDF Structure:	Vdata

- Vdata Name:	“NISTARView”

Vdata Class:	“Geolocation”

Record Size:

Interlace Type:	FULL_INTERLACE

Maximum TBD Records per table.

Table 3-46  NISTAR view indication.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| NISTAREpoch | DFNT_UINT64 | 1 | JulianDay | 2451544… 2465442 | Time of the Ephemeris data used in the calculation (J.D.). |
| NISTARView | DFNT_INT32 | 1 | N/A | 1…10 | An integer representing what object(s) is in the NISTAR field of view. |

### Spacecraft-Sun Angle

- Description:	The angle between the Sun and the spacecraft as viewed from the Earth center.  In other words, this is the Triana-Earth-Sun angle.  This angle is used in calculating the “gibbous effect” one gets when observing the planet from a direction that is different from the direction in which it is lit.  NOTE:  We have allowed a range of 0…/2 for the Sun-Earth-Triana angle to admit data taken in early orbit.

- Vgroup:	“Geolocation”

- HDF Structure:	Vdata

- Vdata Name:	“SpacecraftSunAngle”

Vdata Class:	“Geolocation”

Record Size:

Interlace Type:	FULL_INTERLACE

1 Record per table.

Table 3-47  Spacecraft Sun angle.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| NISTAREpoch | DFNT_UINT64 | 1 | Julian Day | 2451544… 2465442 | Time of first solar angle used in interpolation to collection time (J.D.). |
| EarthSolarAngle | DFNT_FLOAT32 | 1 | Julian Day | 0…/2 | Triana-Earth-Sun angle—this angle should always be less than about 15 degrees |

## Metadata

Each file shall have a global attribute called “metadata” attached to it.  This is an HDF attribute, not a Vdata.  The metadata attribute shall contain information about the product.  It is a single character string with each name=value parameter pair delimited by a “;<CR>” character set.  The <CR> character is defined as ASCII code 13 (hexadecimal).

The values in the latitude and longitude fields shall be the geographic coordinates of the specified pixels in the Earth image.  The centroids of the images are defined as the center of the Earth disk as it appears in the image.

The HDF function Sdsetattr in the C language API or its equivalent, the sfscatt function in the Fortran 77 API, shall be used to assign values to the metadata attribute.  The attribute shall be named “metadata”.  The sd_id argument to the function shall be the file handle.

### NISTAR Level 1 Product Metadata

Description:	A textual metadata.

HDF Structure:	User-defined HDF SD Attribute

Object Name:	“metadata”

Table 3-48  Level 1 product metadata.

| Field Name | Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| Producer_granule_id | String | 30 | N/A | N/A | The name of the HDF file (no null terminator at the end of string). |
| Date | String | 19 | N/A | N/A | yyyy-mm-dd_hh:mm:ss date/time (UTC) of first product |
| Granule_version | Integer (2 digits) | 2 | N/A | 01…99 | The processing version number of the product. |
| Comment | String | 40 | N/A | N/A | A miscellaneous text comment on the product. Null value = “NULL”. Default value is “NULL”. |
| Centroid_latitude | String | 6 | degrees | -90.00… +90.00 | The latitude of the image centroid. E.g., +37.25. Null value = “NULL”. |
| Centroid_longitude | String | 7 | degrees | -180.00… +180.00 | The longitude of the image centroid. E.g., -173.28. Null value = “NULL”. |
| Percent_data_available | Integer (2 digits) | 2 | percentage | 0…100 | Indicates the percentage of data expected in a 24-hour interval actually available in the product. |
| Data_quality | String | 4 | N/A | GOOD or BAD | Indicates if the quality of the data in the product is good enough for scientific analysis (GOOD) or not (BAD). |

### Metadata Text Format

Producer_granule_id=name; <CR>

Date=yyyy-mm-dd_hh:mm:ss;<CR>

Granule_version=xx;<CR>

Comment=NULL;<CR>

Centroid_latitude=+/-xx.xx;<CR>

Centroid_longitude=+/-xx.xx;<CR>

Percent_data_available=xxx;<CR>

Data_quality=GOOD/BAD;<CR>

# The Level 2 Products

## Product Overview

The NISTAR level 2 data products are generated from the level 1 products.  The level 2 science data consist of irradiance values for nearly the entire Earth.  The irradiance of the current full-Earth view of the Earth is measured every ten minutes for the full 24-hour observation interval.  Lunar irradiance values contain information only about the side of the Moon that was visible to the spacecraft during the 24-hour interval.  Level 2 products do not contain star field data.  The data are collected and stored on ten minute, one hour, one day, one week, one month, and yearly time scales.  The ten-minute samplings are actual irradiance measurements taken at the moment.  The larger time interval samplings are TBD averages (?) of the ten minute samplings collected over the previous hours.

## Irradiances

These quantities are calculated on several time scales: 10 minutes, hour, day, week, month, and year.  The radiometer readings (irradiance measurements) are taken every second.  While these measurements are being taken, the shutter is opened and closed every five minutes.  The result is a (slightly "noisy") square wave with a period of five minutes.  An accurate extraction of the Earth's irradiance from the data requires an accurate measurement of the amplitude of this square wave.  Modern techniques for carrying out this analysis involve performing a Fourier transform on the data and extracting the wave, which has a frequency at the fundamental frequency (in this case, five minutes).  The amplitude of this Fourier component has a direct relationship to the height of the square wave, and hence to the Earth's irradiance.

### Earth Irradiances

These data are the irradiance values computed from level 1 data collected while the instrument was aimed at the Earth.  The data are collected at ten minute intervals and are averaged for larger intervals of time.

#### Measurements at Ten Minute Resolutions

- Description:	This object contains the Earth irradiances as measured by the four detectors at ten

- minute samplings.

- HDF Structure:	Vdata

- Vdata Name:	“EarthIrradiances_10min”

Vdata Class:	“Irradiances”

Interlace Type:	FULL_INTERLACE

~144 Records per table, one for each 10 minute period.

Table 4-49  Earth irradiances at 10 minute resolution.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| Hour | DFNT_UINT16 | 1 | hours | 0…23 | This is the hour of the current day within which this sample was taken. |
| Minutes | DFNT_UINT16 | 1 | minutes | 0…59 | The minutes within the current hour at which the sample was taken. |
| EarthSolarAngle | DFNT_FLOAT32 | 1 | radians | 0…/6 | Triana-Earth-Sun angle—this angle should always be less than about 15 degrees |
| SubsatelliteLatitude | DFNT_FLOAT32 | 1 | radians | -… | The noon-time latitude of the subsatellite point |
| SubsatelliteLongitude | DFNT_FLOAT32 | 1 | radians | -/2…/2 | The noon-time longitude of the subsatellite point |
| Radiometer1 | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance reading of first Radiometer |
| Radiometer2 | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance reading of second Radiometer |
| Radiometer3 | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance reading of third Radiometer |
| Photodiode | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance reading of photodiode |

#### Average Measurements at One Hour Resolutions

- Description:	This object contains averages of the Earth irradiances as summed over a given hour.

- HDF Structure:	Vdata

- Vdata Name:	“EarthIrradiances_hourly”

Vdata Class:	“Irradiances”

Interlace Type:	FULL_INTERLACE

~24 Records per table, one for each hour of the day.

Table 4-50  Earth irradiances at 1 hour resolution.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| Hour | DFNT_UINT16 | 1 | Hours | 0…23 | This is the hour of the current day for which this value was computed. |
| EarthSolarAngle | DFNT_FLOAT32 | 1 | JulianDay | 0…/6 | Triana-Earth-Sun angle at 30 minutes past the current hour—this angle should always be less than about 15 degrees |
| SubsatelliteLatitude | DFNT_FLOAT32 | 1 | Radians | -… | The latitude of the subsatellite point at 30 minutes past the hour. |
| SubsatelliteLongitude | DFNT_FLOAT32 | 1 | Radians | -/2…/2 | The longitude of the subsatellite point at 30 minutes past the hour. |
| Radiometer1 | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the first radiometer |
| Radiometer2 | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the second Radiometer |
| Radiometer3 | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the third radiometer |
| Photodiode | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the photodiode |

#### Average Long-term Measurements

- Description:	This object contains averages of the Earth irradiances for the current day, week, month,

- and year.

- HDF Structure:	Vdata

- Vdata Name:	“EarthIrradiances_summary”

Vdata Class:	“Irradiances”

Interlace Type:	FULL_INTERLACE

Maximum 1 Record per table.

Table 4-51  Earth irradiances long-term summary.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| Radiometer1_day | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the current 24-hour period for the first radiometer |
| Radiometer2_day | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the current 24-hour period for the second radiometer |
| Radiometer3_day | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the current 24-hour period for the third radiometer |
| Photodiode_day | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the current 24-hour period for the photodiode |
| Radiometer1_week | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the current week (beginning with Monday) for the first radiometer |
| Radiometer2_week | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the current week (beginning with Monday) for the second radiometer |
| Radiometer3_week | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the current week (beginning with Monday) for the third radiometer |
| Photodiode_week | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the current week (beginning with Monday) for the photodiode |
| Radiometer1_month | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the current month (beginning with the first day of the month) for the first radiometer |
| Radiometer2_month | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the current month (beginning with the first day of the month) for the second radiometer |
| Radiometer3_month | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the current month (beginning with the first day of the month) for the third radiometer |
| Photodiode_month | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the current month (beginning with the first day of the month) for the photodiode |
| Radiometer1_year | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the current year (from 01 January to the current day) for the first radiometer |
| Radiometer2_year | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the current year (from 01 January to the current day) for the second radiometer |
| Radiometer3_year | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the current year (from 01 January to the current day) for the third radiometer |
| Photodiode_year | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the current year (from 01 January to the current day) for the photodiode |

### Lunar Irradiances

These data are the irradiance values computed from level 1 data collected while the instrument was aimed at the Moon.  The data are collected at ten minute intervals and are averaged for larger intervals of time.

#### Measurements at Ten Minute Resolutions

- Description:	This object contains the Lunar irradiances as measured by the four detectors at ten

- minute samplings.

- HDF Structure:	Vdata

- Vdata Name:	“LunarIrradiances_10min”

Vdata Class:	“Irradiances”

Interlace Type:	FULL_INTERLACE

Maximum 144 Records per table, one for each 10 minute period.

Table 4-52  Lunar irradiances at 10 minute resolution.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| Hour | DFNT_UINT16 | 1 | Hours | 0…23 | This is the hour of the current day within which this sample was taken. |
| Minutes | DFNT_UINT16 | 1 | Minutes | 0…59 | The minutes within the current hour at which the sample was taken. |
| MoonSolarAngle | DFNT_FLOAT32 | 1 | JulianDay | 0…/6 | Triana-Moon-Sun angle—this angle should always be less than about 15 degrees |
| LunarSubsatelliteLatitude | DFNT_FLOAT32 | 1 | Radians | -… | The Lunar latitude of the subsatellite point (selenagraphic coordinates) |
| LunarSubsatelliteLongitude | DFNT_FLOAT32 | 1 | Radians | -/2…/2 | The Lunar longitude of the subsatellite point (selenagraphic coordinates) |
| Radiometer1 | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance reading of first Radiometer |
| Radiometer2 | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance reading of second Radiometer |
| Radiometer3 | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance reading of third Radiometer |
| Photodiode | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance reading of photodiode |

#### Average Measurements at One Hour Resolutions

- Description:	This object contains averages of the Lunar irradiances as summed over a given hour.

- HDF Structure:	Vdata

- Vdata Name:	“LunarIrradiances_hourly”

Vdata Class:	“Irradiances”

Interlace Type:	FULL_INTERLACE

Maximum 24 Records per table, one for each hour of the day.

Table 4-53  Lunar irradiances at 1 hour resolution.

| Field Name | HDF Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| Hour | DFNT_UINT16 | 1 | Hours | 0…23 | This is the hour of the current day for which this value was computed. |
| MoonSolarAngle | DFNT_FLOAT32 | 1 | Julian Day | 0…/6 | Triana-Moon-Sun angle at 30 minutes past the current hour—this angle should always be less than about 15 degrees |
| LunarSubsatelliteLatitude | DFNT_FLOAT32 | 1 | Radians | -… | The lunar latitude (selenagraphic coordinates)of the subsatellite point at 30 minutes past the hour. |
| LunarSubsatelliteLongitude | DFNT_FLOAT32 | 1 | Radians | -/2…/2 | The lunar longitude (selenagraphic coordinates)of the subsatellite point at 30 minutes past the hour. |
| Radiometer1 | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the first radiometer |
| Radiometer2 | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the second Radiometer |
| Radiometer3 | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the third radiometer |
| Photodiode | DFNT_FLOAT32 | 1 | Wcm2 | 0…1E-5 | Irradiance summary for the photodiode |

## Metadata

Each file shall have a global attribute called “metadata” attached to it.  This is an HDF attribute, not a Vdata.  The metadata attribute shall contain information about the product.  It is a single character string with each name=value parameter pair delimited by a “;<CR>” character set.  The <CR> character is defined as ASCII code 13 (hexadecimal).

The values in the latitude and longitude fields shall be the geographic coordinates of the specified pixels in the Earth image.  The centroids of the images are defined as the center of the Earth disk as it appears in the image.

The HDF function Sdsetattr in the C language API or its equivalent, the sfscatt function in the Fortran 77 API, shall be used to assign values to the metadata attribute. The attribute shall be named “metadata”.  The sd_id argument to the function shall be the file handle.

### NISTAR Level 2 Product Metadata

Description:	A textual metadata.

HDF Structure:	User-defined HDF SD Attribute

Object Name:	“metadata”

Table 4-54  Level 2 product metadata.

| Field Name | Data Type | Order | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| Producer_granule_id | String | 30 | N/A | N/A | The name of the HDF file (no null terminator at the end of string). |
| Current_date | String | 19 | N/A | N/A | yyyy-mm-dd_hh:mm:ss date/time (UTC) of the current day. |
| Begin_date | String | 19 | N/A | N/A | yyyy-mm-dd_hh:mm:ss date/time (UTC) of the beginning day from which level 2 data was tabulated. |
| End_date | String | 19 | N/A | N/A | yyyy-mm-dd_hh:mm:ss date/time (UTC) of the final day up to which level 2 data was tabulated. Normally, this day will be the same as the Current_date. |
| Calibration_version | String | TBD | N/A | N/A | The name of the calibration file used to calibrate the raw instrument data combined to create the level 2 products. |
| Granule_version | Integer (2 digits) | 2 | N/A | 01…99 | The processing version number of the product. |
| Comment | String | 40 | N/A | N/A | A miscellaneous text comment on the product. Null value = “NULL”. Default value is “NULL”. |
| Centroid_latitude | String | 6 | degrees | -90.00… +90.00 | The latitude of the image centroid. E.g., +37.25. Null value = “NULL”. |
| Centroid_longitude | String | 7 | degrees | -180.00… +180.00 | The longitude of the image centroid. E.g., -173.28. Null value = “NULL”. |
| Percent_data_available | Integer (2 digits) | 2 | percentage | 0…100 | Indicates the percentage of data expected in a 24-hour interval actually available in the product. |
| Data_quality | String | 4 | N/A | GOOD or BAD | Indicates if the quality of the data in the product is good enough for scientific analysis (GOOD) or not (BAD). |

### Metadata Text Format

Producer_granule_id=name; <CR>

Current_date=yyyy-mm-dd_hh:mm:ss;<CR>

Begin_date=yyyy-mm-dd_hh:mm:ss;<CR>

End_date=yyyy-mm-dd_hh:mm:ss;<CR>

Calibration_version=xxxxxxxxxx;<CR>

Granule_version=xx;<CR>

Comment=NULL;<CR>

Centroid_latitude=+/-xx.xx;<CR>

Centroid_longitude=+/-xx.xx;<CR>

Percent_data_available=xxx;<CR>

Data_quality=GOOD/BAD;<CR>